*********************************************************

*		       >>>>>>> NOTE <<<<<<<		    *

*							    *

*		  THIS IS A PRELIMINARY DOCUMENT	    *

*	  THE DSP STARTERS KIT IS NOT YET PRODUCTIZED	    *

*							    *

*********************************************************

DSP STARTERS KIT

( D S K )

USERS GUIDE AND TECHNICAL REFERENCE

DSK HARDWARE AND SOFTWARE LICENSE

---------------------------------

Hardware

Your unmodified DSK circuit board has been carefully tested and

inspected in the factory for functionality.  Be sure to check the

board yourself BEFORE making any changes!  If you do, this will

invalidate any kind of warranty, and we will not accept the board

back for repair.

Software

DSKD.EXE (tm) TMS320C26 RS232 Monitor and Debugger   Version 1.xx

Copyright 1992 Texas Instruments, Inc.  All rights reserved.

DSKA.EXE (tm) TMS320C2x DSP Designers Kit Assembler Version 1.02

Copyright 1992 Texas Instruments, Inc.  All rights reserved.

The software you have received with your DSK can be freely copied

and distributed IF:

- No Fee is charged for use, copying or distributing

- It is not modified in any way

Use of the DSK software does not require any specific registration

fee.  DSK software is distributed freely as public domain software

from the TMS320 BBS (713)274-2324 (N-8-1, 1200,2400 and 9600 baud).

If you have comments, looking for new applications or wish to trade

hardware and software ideas the BBS is an ideal place to do it.

Texas Instruments hereby disclaims all warranties relating to this

software, whether express or implied, including without

limitation any implied warranties of merchantability or fitness

for a particular purpose. Texas Instruments will not be liable for any

special, incidental, consequential, indirect or similar damages

due to loss of data or any other reason, even if Texas Instruments or an

agent of Texas Instruments has been advised of the possibility of such

damages.  In no event shall Texas Instruments's liability for any damages

ever exceed the price paid for the license to use software,

regardless of the form of the claim.  The person using the

software bears all risk as to the quality and performance of the

software.

Introduction to DSP Designers Kit

---------------------------------

OVERVIEW

Your DSK development kit is designed to be operated from an IBM PC/AT

or compatible.  The user interface is through an RS-232 serial communications

link.  Provisions have been made in the hardware specification making it

possible to upgrade the DSK module with extra memory and features of your

own design.

KIT CONTENTS: Your kit should contain the following materials.

- DSK circuit board module

- TMS320C26 DSP processor

- TLC32040 Analog Interface Circuit (AIC)

- RS-232 interface chips

- AC/DC Power supply circuitry

- Floppy disk containing

- DSKD.EXE    - The debugger interface for the DSK module

- DSKA.EXE     - A TMS320C2x assembler

- SPECTC26.ASM - A spectrum analyzer application for the DSK module

- SPECTTWI.ASM - FFT twiddle tables

- SPECTC26.DSK - The executable object of SPECTC26.ASM

- C26BFNL.ASM  - Assembler source of the TMS320C26BFNL's bootload code

- C26BFNL.DOC  - Document describing bootloader operation

- ?????        - Check the subdirectories!  New applications will be

added as they become available!

- DSK Users Guide and Technical Reference

NOTE: If you received the software, either from the BBS or a friend, the

DSK board can not be purchased seperately.  However, the interface

is simple and you might want to consider making one yourself!

=======================================================================

WHAT YOU WILL NEED TO PROVIDE

-----------------------------

Your DSK circuit board and software has been designed to be operated

from an IBM PC/AT running DOS 4.01 or higher.  Your computer must have

an RS-232 asynchronous port installed to run the DSK.  Also, a suitable

9VAC (UL Class II) wall mount transformer with a 2.1mm (2.55mm in early

versions) pin diameter power jack.  The outer diameter is 5.50mm.  A

chart showing the DSK connector and corresponding plugs is given below.

+----------+--------+--------+

|          | DSK    | XFMR   |

|          | CONN   | PLUG   |

+----------+--------+--------+

New | PIN   ID | 2.00mm | 2.10mm |

| SHAFT OD | 5.55mm | 5.50mm |

+----------+--------+--------+

Old | PIN   ID | 2.50   | 2.55mm |

| SHAFT OD | 5.55   | 5.50mm |

+----------+--------+--------+

RS232 PORT AND CABLES

---------------------

To run the DSK you will need to have an RS-232 port available.  If you

do not, or the one you have is currently being used (possibly a mouse), you

may need to purchase an Asynchronous communications for your PC or possibly

an RS-232 upgrade kit.  Consult your PC manual for more details.

You will also need to purchase an RS-232 cable to connect to the DSK module.

To save space the DSK was designed with a DB9 RS-232 connection.  This

connector style is very popular with newer PC's and laptop computers.  If

you do not have the neccessary cables, they can be purchased from a local

computer store.  If you are making your own cable, the DSK module connections

as shown below.

DSK CIRCUIT BOARD

..........................................

:         RS232 LINE    C26 DSP          :

-----+             :    / |    BUFFERS     +----+           :

|TX   (XMIT)  :3 /   |  |\            |    |           :

PC/AT|---------------|    |--|  >o---------|BIO |  +-----+  :

HOST |TR   (RCV)   :2|   R|  |/  /|        |    |--|     |--- AIN

|---------------| D S|---o<  |--------|XF  |--| TLC |  :

ASYNC|DTR  (/RS)   :4| B 2|  |\  \| |\     |    |--|32040|  :

SER  |---------------| 9 3|--|  >o--|  >o--|/RS |--| AIC |--- AOUT

PORT |GND          :5|   2|  |/     |/     |    |--|     |  :

|---------------|    |----------------|GND |  +-----+  :

|             :  \   |                |    |           :

-----+             :    \ |                +----+           :

:                       +-----------+    :

:       +12(unreg) -----| Half Wave |    :

:               +5 -----| rectify   |----- 9 VAC

:               -5 -----| +/-5V reg |    :

:       -12(unreg) -----|           |    :

:                       +-----------+    :

..........................................

POWER SUPPLY

------------

You will need to a 9 VAC (>250 mA) supply to run the DSK.  The DSK module

accepts a 2.1mm (2.5mm early revs) power jack, common to many wall mount AC

transformers.  A low current UL listed transformer is recommended for the DSK

module since they are designed to withstand most power faults for a brief

period of time.  If you do have problems, immediately unplug the transformer

from the wall and check that you have everything set up right. In the United

States a suitable transformer (9 VAC, 1.3A) can be aquired from Radio Shack

(Cat. No. 273-1611A).  However, if you have a early revision board you will

also want to replace the 2.1mm jack on this transformer.

If you decide to use the DSK's on board power supply and regulators for

external circuits, be sure to keep the loading to less than about 50 mA.

Regulation for the DSK is minimal and can not handle a lot of additional

current.

The DSK module can also be run from a suitable +/-7 to +/-12 VDC supply.

In this case the minimum voltage is set by the lower regulation limit of

the onboard +/-5 V regulators and the upper limit by the maximum allowable

RS232 level.  To do this, power is directly applied to the double row header

where all signals have been made available.  Note: Be careful to connect

the power supply correctly since the DSK is not warrentied after modifications

have been made.  Failure to do this can also cause significant damage to

the DSK module.

SETTING UP THE DSK SOFTWARE

---------------------------

It is strongly recommended that you first copy the contents of the DSK

distribution disk to a safe place for backup.  This can be done by using the

DOS DISKCOPY utility.  It is not recommended that you use the DSK from the

distribution diskette.

Next, we need to copy the DSK tools to a directory which has been set

aside for DSK use.  Since the DSKD debugger is also compatible with

TI's COFF assemblers and linkers you can also copy everything to that directory

to keep things simple.  Next you should modify your PC's path statement to

make the DSK tools available from other directories.

The DSK software you will need primarily consists of two executable files,

DSKD.EXE and DSKA.EXE.  These two files are the debugger and assembler

respectively.  The other files contained in the distribution disk are

for examples and development support.

When the DSKD is first started it also creates some configuration and

parameter file (DSKD.CFG and DSKD.PRM).  They can be modified to customize

the colors that the debugger uses.  If something goes wrong, delete the

old ones and start over.

=============================================================================

DSKD

-----

INTRODUCTION

DSKD is a debugger and user interface designed to be used with the RS232

Bootload option of the TMS320C26 chip.  Features of DSKD include an easy to

use windowed display of program, data, internal registers and status.  The

user interface is capable of loading and executing user code with single

step, breakpoint and run-time halt capabilities.  The full featured DSP code

which is responsible for the host communications has been kept to a very

compact 256 words in program RAM B0 making it possible to run reasonably

complicated code completely on-chip while still operating the debugger.

INPUT FILES FOR DSKD

Valid code formats for DSKD include both DSK output files from the DSKA

assembler and COFF format files from the TMS320 DSP Fixed Point Assembler.

If you need features not found in the DSKA assembler, contact your local

TI sales representitive for purchasing information on the COFF assembler.

HOW DSKD WORKS

DSKD works in conjunction with the RS232 bootload option of the

TMS320C26.  When the 'C26 is reset, the on-chip bootload ROM checks the

status of the bit I/O (BIO) input pin.  If the BIO pin is high and no

externaly valid bootload EPROM is found, then the chip enters the

RS232 bootload option.

When the BIO line goes low, this indicates that an RS232 transmission

start bit has been sent by the host.  Since the baud rate is unknown, the

'C26 begins counting the number of elapsed CPU cycles untill the BIO

line goes high again at the end of the start bit.  The baud rate is set

by the width of the start bit.  That is, the first data bit must go high!

Following baud rate detection, some configuration words are transmitted

followed by code from the host and finally a sumcheck and dummy word.  If

you are interested in this protocol, it is explained in the C26BFNL bootloader

document and bootloader source code.

The bootloader locates code into on-chip RAM beginning with B0.  In the

case of the DSK a debugging and communications kernal is loaded which is

capable of bi-directional RS232 communications using XF and BIO as well as

debugging using the TRAP mneumonic and INT2 (incoming RS232 message).  When

a debug interrupt occurs, the DSK kernal also saves and restores the internal

registers.  This should not concern a DSK user so long the kernal is not

corrupted.  If it is corrupted, the the DSKD (and the DSP) will not operate

properly.

>>>>>>  IMPORTANT <<<<<<

Your DSK module uses the INT2 line tied to the BIO input to indicate that

there is activity on the RS232 receive line.  If you turn of the global

interrupt enable bit or unmask INT2 the runtime halt feature will not work.

Likewise, if you rewrite the vector table for either TRAP or INT2 the debugger

kernal will fail to operate correctly.

RESERVED FOR DSKD

> All of RAM block B2                 CPU register storage

> First 256 words of B0               debugger communications kernal

> B0 configured as program RAM                   ""

> C26 must be in uC mode              RS232 receive is in ROM

> INT2                                Halts running CPU

> TRAP opcode                         Breakpoints and single step

INTERRUPT VECTOR TABLE

When writing code for the DSK, all interrupts are directed to a primary

jump table in ROM followed by a secondary table in RAM which can be modified

to the users needs.  BE CAREFUL WITH ARP MODIFICATIONS!  The primary vector

table modifies the ARP before the ISR has been started.  If you need to

recover the old ARP, it is in ST1 not ST0!

.ps     0000h

RESET:  B       BOOT  ,*,AR7  ;The primary vector table in ROM directs

B       B0INT0,*,AR0  ;interrupts to RAM B0 where the user can

B       B0INT1,*,AR0  ;define a branch to his or her own code.

B       B0INT2,*,AR0  ;

.space  0100h ;bits   ;RESERVED

B       B0TINT,*,AR0  ;Since the ARP has been modified, it is

B       B0RINT,*,AR0  ;neccesary to recover the ARP from the

B       B0XINT,*,AR0  ;ARB in STAT1 whenever an interrupt is

B       KERNAL,*,AR0  ;processed.

.........................................

BOOT:   ; Bootloader code                       ;

.........................................

||      ||           ; rest of ROM

B      B0USER        ; Bootloader exits to user code in B0

||      ||           ;

;--------------------------------------------------------------

UNUSED PROGRAM SPACE  (00100h - 0fa00h)

;--------------------------------------------------------------

.ps     0fa00h        ;  >>>>> AUXILIARY VECTOR TABLE <<<<<<

B0USER: B       START         ;

B0INT0: B       INT0          ;the bootloader ROM performs auxiliary

B0INT1: B       INT1          ;register pointer modification.

B0INT2: B       INT2          ;

B0TINT: B       TINT          ;Since the ARP has been modified, it is

B0RINT: B       RINT          ;neccesary to recover the ARP from the

B0XINT: B       XINT          ;ARB in STAT1 whenever an interrupt is

;---------------------------------------------------------------

KERNAL: ;                     ; NOTE: the debugger kernal starts at the

;  apx. 256 words     ;       TRAP vector in B0 program

;                     ;

;---------------------------------------------------------------

.ps    0fb00h        ;Your code can start here

.entry               ;

START:   larp   0             ;

lark   AR0,0         ;

lack   03fh          ;Turn on all interrupts

ldpk   0             ;

sacl   04h           ;

SELF:    B      SELF          ;

;---------------------------------------------------------------

; Simple interrupt processing (quick return)

INT0:                         ;

INT1:                         ;

INT2:                         ;

TINT:                         ;

RINT:                         ;

XINT:   ldpk   0              ;recover the ARP from ST1

sst1   060h           ;BEFORE returning to your code!

lst1   060h           ;

ret                   ;

.end                  ;

NOTE: If you are building your own DSK module (or a DSP system), you may

have an older version of the bootloader TMS320C26FNL which operates

different from the TMS320C26BFNL.  Several bugs were fixed, including

RS232 autobaud, as well as a new EPROM bootload option.  The DSKD

has an automatic detection mechanism built in, but several tries may be

needed to establish a good link.

DSKD COMMANDS AND HELP MENU

For quick reference, a list of commands can be viewed from within DSKD by

hitting 'H' (Help) while at the command line.  This list can also be sent to

a file while in the Help menu by hitting the 'F' (File) key which may be

helpful for future revisions.  The following is the output listing.

Help print at 09:25:23 on 27.1.1993 for version X.XX

D S P    S T A R T E R     K I T    D E B U G G E R

���������������������������������������������������������������

Usage of the program: dskd [[-|\][options]

Options: Please note that options aren't case significand

��������

? or H : this display

E      : defines with 4 following digits the entry point

I      : selects logic level for DTR->Reset (default->inverse)

M[0..3]: set confmode 0,1,2,3 [default=1]

S      : selects the default screen length (25)

L      : selects the EGA/VGA screen length (43 or 50)

COM1,2 : comport 1 [default] or 2 (3 and 4 aren't supported)

optinonal definition: 'C1' or 'C2' can be used

Bxxxxx : xxxxx selects the baudrates! Working as today:

4800,9600,19200,38400

Function Key definitions

������������������������

F1     Help Information

F2     Print Screen to 'screen.srn'

F3     Display Directory

F4     unused

F5     execute to breakpoint --> same as XG

F6     unused

F7     unused

F8     singlestep --> same as F10, XS, blank, return

F9     unused

F10    execute over function --> same XF

F11    Reverse Assembler window

sF1    Reverse Assembler window

F12    Trace ON/OFF

sF2    Trace ON/OFF

sF2 .. sF12: User defined functions (editor, assembler,...)

--> not implemented yet!!!!!

BA     Breakpoint add

BD     Breakpoint delete

BE     Breakpoint enablle/disable

BL     Breakpoint list

C      Copy/Move

CDD    Copy Data block to another Data area

CDP    Copy Data block to Program area

CPD    Copy Progrom block to Data area

CPP    Copy Progrom block to another Program area

DD     Display Data memory

DB     Display Breakpoints

DF     Display Format: see subcommands

DFB    Display Data in format big (expo=16,mantissa=32/Q30)

DFC    Display Data in format character

DFD    Display Data in format double

DFE    Display Data in format short float (exp=16,mant=16/Q14)

DFT    Display Data in format long float (exp=32,mant=64/Q62)

DFF    Display Data in format float

DFI    Display Data in format integer

DFL    Display Data in format long

DFO    Display Data in format octal

DFP    Display Data in format packed string

DFU    Display Data in format unsigned integer

DFQ    Display Data in format signed Q15

DFS    Display Data in format string

DFU    Display Data in format unsigned integer

DFX    Display Data in format hexadecimal

DM     Display Memory Window: see subcommands

DMA    Display Memory Window: set Address

DMB    Display Memory Format: big (expo=16,mantissa=32/Q30)

DMD    Display Memory Format: double (IEEE)

DME    Display Memory Format: short float (exp=16,mant=16/Q14)

DMT    Display Memory Format: long float (exp=32,mant=64/Q62)

DMF    Display Memory Format: float (IEEE)

DMI    Display Memory Format: integer

DML    Display Memory Format: long

DMO    Display Memory Format: octal

DMQ    Display Memory Format: Q15

DMU    Display Memory Format: unsigned int

DMX    Display Memory Format: to hexadecimal

DP     Display Program memory

DV     Display Version

DS     display Status register information

FD     Fill Data Memory with a value

FP     Fill Program Memory with a value

Help   Help display

I      Initialize to the entry point of the program

L      Load data (program, [formatted] data from a file)

LC     Load Coff file: common object file format

LD     Load DDK file : DDK file format

LR   # Load Register

LF     Load Format

LFP    Load Format Program: 4 hex digits per line

LFX    Load Format Hex to data: 4 hex digits per line

LFI    Load Format integer: 1 int per line

LFU    Load Format unsigned to data: 1 unsigned int per line

LFQ    Load Format Q15 to data: 1 Q15 number per line

LFL    Load Format long to data: 8 hex digits per line

LFF    Load Format Float to data: 1 IEEE float number per line

LFD    Load Format Double to data: 1 IEEE double number per line

LFE    Load Format Efloat to data: 1 special float per line

LFB    Load Format Bfloat to data: 1 big float per line

LFT  # Load Format Tfloat to data: 1 long float per line

LFC  # Load Format Char to data: 1 char ver line

LFS  # Load Format packed String to data: String[100] on line

LP     Load Program counter: done from ModifyRegister display

MR     Modify 320C26 Register: PC,ACCU,ARx,.....

MP     Modify Program Memory

MD     Modify Data Memory

MI   # Modify Input-port (read value)

MO   # Modify Output-port (write value)

Q      Quit (or ctrl Q)

R      Reset the 320C26

S      Save (program, [formatted] data to a file)

SD     Save Data memory to file

SF     Save Format

SFP    Save Format Program: 4 hex digits per line

SFX    Save Format Hex: 4 hex digits per line

SFI    Save Format integer: 1 int per line

SFU    Save Format unsigned: 1 unsigned int per line

SFQ    Save Format Q15: 1 Q15 number per line

SFL    Save Format long: 8 hex digits per line

SFF    Save Format Float: 1 IEEE float number per line

SFD    Save Format Double: 1 IEEE double number per line

SFE    Save Format Efloat: 1 special float per line

SFB    Save Format Bfloat: 1 big float per line

SFT  # Save Format Tfloat: 1 long float per line

SFC  # Save Format Char: 1 char ver line

SFS  # Save Format packed String to data: String[100] on line

SR   # Save all CPU Register to file: not implemented yet

SP     Save Program memory to file

V      Verify Flag ON/OFF - default=ON for(Load ...)

W      Watch Window definition

WA     Watch window: Address definition

WD     Watch window: Delete watch number

WM     Watch window: Modify address of watch

WF     Watch window Format: see subcommands

WFB    Watch window Format: big (exp=16,mant=32/Q30)

WFT    Watch window Format: long (exp=32,mant=64/Q62)

WFD    Watch window Format: double

WFE    Watch window Format: short float (exp=16,mant=16/Q14)

WFF    Watch window Format: float

WFI    Watch window Format: integer

WFP    Watch window Format: packed string

WFQ    Watch window Format: Q15 (sign . fraction)

WFS    Watch window Format: string

WFU    Watch window Format: unsigned integer

XA     eXecute an go to Address: define a destination Adress

XF     eXecute a function: CALL or CALA

XG     eXecute Go to Breakpoint

XL     eXecute and Leave the debugger

XN     eXecute 'n' single steps

XS     eXecute one single step

XC     eXecute with calling a function (includes stack handling)

XR     eXecute and Free Run (ignore switching XF)

see: F8, F10, blank, return

# this symbol in column 6 means:

function not implented yet!

Floating Point formats:

�����������������������

.Float   32 bit IEEE standarized floating point format

.Double  64 bit IEEE standarized double floating point format

.Bfloat  'big float' / 16 bit exponent + 32 bit mantissa

exponent: 2's complement / signed mantissa = Q30

.Tfloat  'long float' / 32 bit exponent + 64 bit mantissa

exponent: 2's complement / signed mantissa = Q62

.Efloat  'enhanced float' / 16 bit exponent + 16 bit mantissa

exponent: 2's complement / signed mantissa = Q14

Register Definitions:

�������������������������������������������

PC    Program counter: 16 bit

ACCU  Accumulator: 32 bit with C (carry) in ST1

PREG  Product Register: 32 bit for 16* 16 bit multiplication

TREG  Temporary Register: 16 bit for multiplaction

and special instrunctions

ARi   Auxiliary Register: 16 bit with n=0..7 used as

counter and pointer

ST0   Status Register 0: 16 bit

ST1   Status Register 1: 16 bit

STCKi Stack Register: 16 bit with n=0..7 used as

Hardware Stack.

Please note that the debugger uses one level stack

for its own purpose!!

DRR   Data receive register at address 0:

16 bit for the serial port

DXR   Data transmit register at address 1:

16 bit for the serial port

TIM   Timer register at address 2: 16 bit

PRD   Period register at address 3: 16 bit

IMSK  Interupt mask register at address 4:

6 bits for masking 6 interrupts

GREG  Global register at address 5:

8 bits to define data memory as gloabal

ARP   ST0: Auxiliary register pointer: 3 bits

ARB   ST1: Auxiliary register pointer buffer: 3 bits

INTM  ST0: Interrupt mode (global int enable) : 1 bit

DP    ST0: Data Page pointer: 9 bits

CNF   ST1: internal Prog/Data configuration: 2 bits

TXM   ST1: FSX mode bit: 1 bit

PM    ST1: Preg to accu shift mode: 2 bits

C     ST1: Carry bit: 1 bit

TC    ST1: Test/Control: 1 bit

FO    ST1: serial port control (8/16 bit mode): 1 bit

HM    ST1: Holdmode selection: 1 bit

Please note that the following is reserved

������������������������������������������

1) IMR=4 and EINT is set from the debugger to stop a

a running program: Please don't override the bit IMR=4,

use logical instructions to set the right interrupt mask!

2) Do not overwrite locations 0xfa00/0xfa01 and 0xfa06/0xfa07.

These are the secondary reset and INT2 vectors

If an IMR bit (other than IMR 4) is set and an interrupt

occurs the single step halts after the ISR has been

executed.  That is, a Single Step TRAP can not be

placed inside the Interrupt routine.

If a breakpoint is set on an interrupt vector (0xfa02...)

and then executed with Single Step.  Interrupts can be

'nested' because EINT is always enabled during Single Step!*

3) Block B2 (data memory 0x60 - 0x7f) is reserved for

debugger use! Don't overwrite this locations!

4) The debugger kernel is located from 0xfa00-0xfaff

in Block B0 configured as Program memory!

5) Confmode 1 or higher is required to configure

Block B0 as program memory! use always CONF 1 or higher.

If not the debugger kernal in B0 can not execute!

6) The instruction TRAP and the memory location 0xfa0e

(trap vector) are reserved for the debugger

Please note also following

�������������������������

Interrupts vectors in the ROM point to Adress 0xfa00 and

higher! Executing User Interrupts (please note that INT2 is

reserved) requires that additional Vectors have to be placed

into the address space 0xfa00-0xfa0d

Please note also that the ARP (auxiliary register pointer) is

set to AR0 with executing the Interrupt vector branch! The

original ARP content is saved to the 3 MSBs in ST1 and can here

extracted and restored!

============================================================================

DSP DESIGNERS KIT ASSEMBLER

---------------------------

The DSP Designers Kit Assembler (DSKA.EXE) is a very simple and easy

to use assembler for the TMS320C2x family of processors.  Only the most

essential features of an assembler have been incorporated to help keep

the assembler as easy to use as possible.  Every effort has been made to

keep the DSKA as compatible as possible to TI's commercialy retailed "TMS320

Fixed Point DSP Assembly Language Tools" which offer many more features.

OVERVIEW

--------

The DSKA assembler differs from many other assemblers in that it does not

go through a linker phase to create an output file.  Instead the DSKA uses

special directives to assemble code to an absolute address 'on the fly'.

The strength of this approach is that small and simple programs can be

created quickly and easily.  Larger executables can also be created by

chaining files together with the '.include' directive.  There is no

provision for linking executables or libraries together at a later time.

COMMAND LINE OPTIONS FOR INVOKING DSKA

--------------------------------------

Useage: DSKA <FILE[.ASM]> [KDL] [asm "code"]

|||   |

|||   pre-assembles 'code' as an include

||+- generate list file FILE.LST

|+-- dis-assemble object to FILE.LST

+--- Keep output. Ignoring ERRORS

FILES OPENED:  FILE.DSK: DSKA Output file for use with DSKD

FILE.OBJ: Temporary object (automaticly deleted)

FILE.INC: File used when using the asm option

Example: To Assemble TEST.ASM to TEST.DSK and generate a listing file TEST.LST

C:\DSK>DSKA TEST.ASM -L

option: -K  If this option is given DSKA will try to generate an output

file irregardless of the outcome of any errors or warnings.

By default an ERROR corrupted file is deleted.

-L  This option generates an intermediate file containing the

unresolved opcodes.  Unresolved symbols are resolved when

the entire file has been read in.  Note that the symbols

are not stored in the intermediate object file and are

therefore not linkable.

-V  Generates a more Verbose listing if the list option is turned

on.  This is not particularily helpful since the added

information is a set of flags which are used by the assembler

and have little meaning in the general sense of debugging.

-D  The disassemble option is most usefull when trying to find

pesky little bugs that seem to come from nowhere.  In this

case a disassembler dissassembles the output file as it is

being created.  Since the opcodes now contain resolved

symbols they are given as absolute values or addresses.

If a ".listoff" directive is encountered, disassembly is

also turned off.  Also, the data space is disassembled.

asm  This option is a feature which allows the user to define

assembler statements from the command line.  Assembler code

in this case is terminated at both ends with a double quote.

The character immediately following the first quote is placed

in the first column.  ASM statements are written to an

include file FILE.INC overwriting the previous file using

that name.

Example:

DSKA  TEST.ASM  L  ASM"FFT .SET 256"  ASM" .entry 0fb00h"

generates the file "TEST.INC" which contains

column 1

|

FFT .SET 256

.entry 0fb00h

DSKA SOURCE FILES:

-----------------

Editors:

DSKA assembler source files can be created with almost any ASCII editor.

Legal characters are any alpha-numeric character, white space and tabs.

What the DSKA can not work with are files created directly from word

processors that use special control characters for formatting.  Usualy

these editors can generate pure ASCII text files but not by default!

Comments:

There are two ways to comment a line with the DSKA assembler.  The first

and prefferable way is to use a semi-colon ";".  A semi-colon can be

placed anywhere in a line and everything after it is ignored.

The second way to add a comment is to use an asterisk "*".  In this case

the asterisk must always be in the first column.  If it is not, it will

be assumed to be part of an opcode and will likely generate an error.

Labels

All labels must begin in the first column and must start with a letter

or an underscore "_".  After the first letter you can then mix in numbers

as you go along.  An optional colon ":" which is usualy needed in other

assemblers as a terminator is not needed.  A label can be up to 8 characters

in length.

When an opcode or directive references a label, the value of the address

where the label is located in memory is substituted.  The only exception

is the LDPK opcode which loads the top nine bits of the address.

Opcodes

Opcodes must begin past the first column or after a white space or tab

character following a label.  The syntax usage of specific TMS320C2x

opcodes can be found in the TMS320C2x Users Guide.  The rules that apply

are given in the examples below.

First, it is usually neccessary to resolve ALL fields in an opcode.  If

an opcode field such as the shift field in a SACL opcode is left out,

the next field found becomes the value for that field and the next field

is not filled in.  If a field is not specified, its value is assumed

to be a null and the opcode is processed accordingly.

Second, expression analysis is NOT performed!  Things like the addition

of two values, multiplies, shifts etc... are not perfomed.  Only direct

substitution of defined variables is allowed.

EXAMPLE

.ps

LAR   AR7,RESULT

LARP  AR7

WRONG: SACL  *,AR7      ; becomes SACL  *,7  (AR7 used for shift)

RIGHT: SACL  *,0,AR7    ; all field specified

OK:    SACL  *          ; last two fields not specified.  This works

.ds

RESULT .word 0

This happens because the numeric value of AR7 becomes 7, which when

placed into the B opcode (in the wrong field) becomes *BR0+ in the

AR modification field.

NOTE: IMPROPERLY SPECIFIED OPCODES DO NOT ALWAYS GENERATE WARNINGS !

==========================================================================

DIRECTIVES

==========================================================================

label  .byte  value1,[value2,]...

The .byte directive places one or more bytes into consecutive memory words.

The 8 MSB's are filled with 0's.  Value can either be numeric or character

constants.  Variable substitution is NOT supported.

LAB1   .byte    -1,2,0Ah,'A'  ;where 'A' is converted to it's ASCII char

==========================================================================

label  .int   value1,[value2,]...

label  .word  value1,[value2,]...

The .int and .word directives place one or more signed 16 bit values into

memory.  Character constants are also legal.  Variable substitution is NOT

supported.

LAB1   .int     0,-1,2,0ABCDh,'B'

==========================================================================

label  .long  value1,[value2,]...

The .long directive places one or more signed 32 bit values or character

constants into memory.  The least significant word is stored first.

LAB1   .long    0,-123456,234567,01234ABCDh,'C'

==========================================================================

label  .float   value1,[value2,]...

label  .double  value1,[value2,]...

label  .efloat  value1,[value2,]...

label  .bfloat  value1,[value2,]...

label  .tfloat  value1,[value2,]...

These directives convert numeric values into various floating point

formats.  The formats are ...

.float    IEEE-784 32 bit floating point format

.double   IEEE     64 bit floating point format

.efloat   This format uses a 16 bit 2's compliment exponent and a 16 bit

2's compliment mantissa.  The mantissa's decimal point is between

bits 14 and 15 making the 1's place explicit.  That is it is not

implied as in IEEE formats where it is normaly suppressed.  In

IEEE this is suppressed to allow for greater precision since the

mantissa is always between 1.0 and 1.9999 in value.  For DSP use

an IEEE number would need to be 'unpacked' before the individual

members are operated on.  As a result speed performance would be

sacrificed.  In the .efloat, .bfloat and .tfloat formats numbers

are left in their unpacked form for greater efficiency.

.bfloat   similar to .efloat except that it has 16 bit exponent and 32 bit

mantissa.

.tfloat   similar to .efloat except that it has 32 bit exponent and 64 bit

mantissa.  NOTE: This is a proposed format for the TMS320C40 and

is only included for compatability.  It is not suitable for a 16

bit processor!

==========================================================================

label  .string value1,[value2,]...

The .string directive places bytes into memory in a packed form in the

order they are encountered.  If a word is not completely filled, the

trailing LSB's are filled with 0's.  Value can either be numeric, character

constant or character string.  Variable substitution is NOT supported.

LAB1   .string  -1,2,0Ah,"ABC",'D'  ;"ABC" is converted to 3 ASCII char's

==========================================================================

label  .space  value

The .space directive is used to insert value bits into the current

segment, either program or data.  Each word in the space is filled

with 0's and words not completely filled are padded with additional

zeros.

Example:

b        past      ;

.space   040h      ;fills in 4 words with 0's

past:   nop                ;

==========================================================================

var   .set    value

The .set directive is used to assign a value to a variable.  This

directive is useful in establishing values throughout the scope of

the assembled code.  For instance, .set could be used to set the

load and program entry addresses or establish values for commonly used

variables like memory mapped registers.

Example:

DRR     .set    0        ; You could include this at the beginning of

DXR     .set    1        ; all of your assembly programs to make it

TIM     .set    2        ; easier to read your code.

PRD     .set    3        ; (You could make this an included file!)

IMR     .set    4        ;

GREG    .set    5        ;

.ps     0fb00h   ; code usually begins here for the DSK

.entry           ;

ldpk    0        ; load Data Page for the MMR's

lalk    0ffffh   ; value to load to the on-chip timer

sacl    TIM      ; This is easy to read

sacl    2        ; This is the same but difficult to read

==========================================================================

.include   "file.ext"

.copy      "file.ext"

These directives are used to chain files together making it easier to

build large programs.  Note that because there is no link phase to

the DSK assembler that multiple definitions of labels and variables

will generate errors and warnings.  Nesting of included files of

up to 8 levels is allowed before terminating the assembler.

Example:

.include        "MMRS.ASM"    ; include a file defining the MMR's

.include        "LINK.ASM"    ; include a file defining the load

; and program entry address

.include        "VECTS.ASM"   ; include a vector table

; BEGIN YOUR CODE HERE

==========================================================================

label   .text

label   .data

These directives select that assembly output either goes to the program

or data space respectively.  The previously used address (program or data

space) is kept in memory making it easy to continue code or data after a

break.

Example

.ps     0fb00h              ;set up load and run address's

.entry                      ;

.include "VECT.ASM"         ;

.ds     0400h               ;

.text

setup:  larp    AR0                 ;setup the CPU in a known state

lark    AR0,0               ;

lark    AR1,0               ;

lark    AR2,0               ;

lark    AR3,0               ;

.data                       ;

vals_1: .int    0,1,2,3,4,5,6,7     ;initialize some integer values

.text                       ;continue with some code

loop:   mar     *+,AR1              ;

mar     *+,AR2              ;

mar     *+,AR3              ;

b       loop,*+,AR0         ;

.data                       ;

vals_2: .float  0,1,2,3,4,5,6,7     ;initialize some floating point values

==========================================================================

.end

The .end directive terminates the assembly process making any code that

follows irrelevent.  Note that an improperly placed .end directive inside

an .include file can also terminate the assembler prematurely.

==========================================================================

.ps     [value]

.ds     [value]

These directives are used to initialize the program and data load

load address's.  They can appear multiple times, making it possible

to selectively not load anything to the gaps that are produced.

Also, if the addresses overlap, it is possible to overwrite code

or data with these directives.  If [value] is ommitted, the last

address is used as in .text or .data.

==========================================================================

.entry  [value]

When this directive is encountered the value given becomes the starting

address of the program counter when the file is loaded.  If no value is

given the current program memory address as determined by the .ps or

.text section becomes the starting address.  If more than one .entry

directive is encountered in the file, the last .entry becomes the starting

address of your code.

Example

.ps     0fb00h      ;

LOOP:   MAR     *+,AR1      ;An infinite loop

B       LOOP,*+,AR0 ;

.entry              ;Start program with initialize

LARP    AR0         ;

LARK    AR0,0       ;

LARK    AR1,0       ;

B       LOOP        ;call the routine

==========================================================================

.liston

.listoff

These directives are sometimes usefull in debugging your code.  They

are used to override the command line -L option which can turn on the

output listing.  The listing is always written to a file whose name is

derived from the original source with an extension of file.LST.

NOTE: .listoff also turns off disassembly since this feature is part

of the LIST file.

==========================================================================

.if       value

.else

.endif

These directives are used to conditionally assemble sections of code.

The .if segment is assembled if value is non-zero.  Value can either

be numeric or a preset (.set) variable.  Note that since DSKA does

not have a link phase and is a one pass assembler, variables must be

initialized BEFORE using them!  Otherwise their value will be zero.

ALso note that expression analysis is not supported.

Example:

ELOOP   .set    1

.if     ELOOP

SELF    B       SELF

.else

NOP

.endif

==========================================================================

.qxx

.lqxx /* 32 bit Q formats */

These directives are used to generate signed 2's compliment fractional

integer and long integer values whose decimal point is displaced xx

places from the LSB.

Example:

.ds      0400h

.Q15     0.25, 0.5, 0.75

<- 15 places ->

0400     00000h  ; #0.000000000000000b

0401     02000h  ; #0.010000000000000b

0402     04000h  ; #0.100000000000000b

0403     06000h  ; #0.110000000000000b

==========================================================================

DSK OUTPUT FILES

The DSK output file supports 16 bit addresses both in program and

data space allowing direct placement of runtime code into a target.

+-- DSK characters:  K designates start of file

|                    1 designates code entry point

|                    9 designates load address

|

|          +-- DSK revision and output file name

|          |                                 +-- Checksum follows

^ |<-------------->|                         |    +-- line terminate

K_DSKA_1.01_TEST.DSK                         |    |

1FB007FB00F                                  ^    ^

9FB00B5588B55A9B55AAB55ABB55ACB55ADB55AEB55AF7AD3CF

9FB08BFFA8BFB01B67E0B60F07C379F

90400M0000M0000M123471234F

:              ^

^              |

|              +-- B designates load to program space

|                  M designates load to data space

|

+-- End of file record

#