HOSTSPEC.ASM

Written By: Keith Larson

TMS320Cxx DSP Applications Engineer

Texas Instruments

A spectrum analyzer using your HOST graphics display!

**************************************************************************

* >>>>>>>>>>>>>>>>>>>>>>>>>>>>>   NOTICE  <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<*

**************************************************************************

*  This code will NOT work with DSKD!                                    *

*  It is designed to work with the DSK loader (DSKL) [G]raphics option.  *

**************************************************************************

QUICKRUN:

To run this program, first assemble it using DSKA. Then start DSKL,

select 'X' for load and execute.  Then select 'G' for graphics.  To exit,

hit any key (except Q) since the keyboard is not flushed and 'Q' will

quit DSKL entirely! to the host via the RS232 link.  It is slower, but

you do not need an oscilliscope!

THEORY OF OPERATION:

Two major changes have been made to the file DSK_SPEC to make HOSTSPEC.

Otherwise, they are essentially identical.

The first is that an RS232 link to the DSKL communications kernal DSK_COMM

has been made.  This is accomplished by making the appropriate call to the

transmit (XMIT) routine in that program.  In this case a call is made to a

jump table in DSK_COMM, making the call easy to manage.  If used in other

programs, you should be careful to save the CPU status BEFORE making the

call as there is no context save and restore performed in DSK_COMM.

The second major change is that the input signal windowing function is

no longer at the input!  Instead, a post convolution of the frequency data

is performed eliminating the need to calculate a raised cosine window

from the twiddle tables.  This is a significant savings in code size and

has allowed the addition of a call to the RS232 communications routines.

The window which is used is raised cosine, which represented in frequency

becomes

Time                           Frequency (Z domain)

w(t) = 1-cos(2*pi*n/N)                 Z[0]                   Z[-2]

W(Z) =  -  ----   +   Z[-1]   -   -----

n=sample                             2                      2

N=number of points

NOTE: This is a 3 tap FIR filter!!!

Very easy and fast!

COMMUNICATING RS232

To keep the DSK and DSKL properly synchronized a handshake protocol is

used where the host (DSKL) must send a word to the DSK before the DSK will

send one back.  Any value can be used, but could in fact be a message to

the DSK to do something else (a future code enhancement!).  The value which

is returned is the frequency domain data, converted to an 8 bit logarithmic

number.  The only restriction is that zero has been reserved as a synch

signal to indicate the beggining of the next frame.

#