1

PM0037                    March, 1984

SPICE VERSION 2G.6 USER'S GUIDE                  _____ _______ ____ ______ _____

(10 AUG 1981)

A.Vladimirescu, Kaihe Zhang,

A.R.Newton, D.O.Pederson, A. Sangiovanni-Vincentelli

Department of Electrical Engineering and Computer Sciences

University of California

Berkeley, CA., 94720

ACKNOWLEDGEMENT: Dr. Richard Dowell and Dr. Sally Liu have ________________

contributed to develop the present SPICE version.  SPICE was

originally developed by Dr. Lawrence Nagel and has been modified

extensively by Dr. Ellis Cohen.

SPICE is a general-purpose circuit simulation program for

nonlinear dc, nonlinear transient, and linear ac analyses.

Circuits may contain resistors, capacitors, inductors, mutual

inductors, independent voltage and current sources, four types of

dependent sources, transmission lines, and the four most common

semiconductor devices: diodes, bjt's, jfet's, and mosfet's.

SPICE has built-in models for the semiconductor devices, and

the user need specify only the pertinent model parameter values.

The model for the bjt is based on the integral charge model of

Gummel and Poon; however, if the Gummel-Poon parameters are not

specified, the model reduces to the simpler Ebers-Moll model.  In

either case, charge storage effects, ohmic resistances, and a

current-dependent output conductance may be included.  The diode

model can be used for either junction diodes or Schottky barrier

diodes.  The jfet model is based on the fet model of Shichman and

Hodges.  Three mosfet models are implemented; MOS1 is described

by a square-law I-V characteristic; MOS2 is an analytical model

while MOS3 is a semi-empirical model.  Both MOS2 and MOS3 include

second-order effects such as channel length modulation,

subthreshold conduction, scattering limited velocity saturation,

small-size effects and charge-controlled capacitances.

1.  TYPES OF ANALYSIS __  _____ __ ________

1.1.  DC ANALYSIS ____  __ ________

The dc analysis portion of SPICE determines the dc operating

point of the circuit with inductors shorted and capacitors

opened.  A dc analysis is automatically performed prior to a

transient analysis to determine the transient initial conditions,

and prior to an ac small-signal analysis to determine the

linearized, small-signal models for nonlinear devices.  If

requested, the dc small-signal value of a transfer function

2

(ratio of output variable to input source), input resistance, and

output resistance will also be computed as a part of the dc

solution.  The dc analysis can also be used to generate dc

transfer curves: a specified independent voltage or current

source is stepped over a user-specified range and the dc output

variables are stored for each sequential source value.  If

requested, SPICE also will determine the dc small-signal

sensitivities of specified output variables with respect to

circuit parameters.  The dc analysis options are specified on the

.DC, .TF, .OP, and .SENS control lines.

If one desires to see the small-signal models for nonlinear

devices in conjunction with a transient analysis operating point,

then the .OP line must be provided.  The dc bias conditions will

be identical for each case, but the more comprehensive operating

point information is not available to be printed when transient

initial conditions are computed.

1.2.  AC SMALL-SIGNAL ANALYSIS ____  __ ____________ ________

The ac small-signal portion of SPICE computes the ac output

variables as a function of frequency.  The program first computes

the dc operating point of the circuit and determines linearized,

small-signal models for all of the nonlinear devices in the

circuit.  The resultant linear circuit is then analyzed over a

user-specified range of frequencies.  The desired output of an ac

small-signal analysis is usually a transfer function (voltage

gain, transimpedance, etc).  If the circuit has only one ac

input, it is convenient to set that input to unity and zero

phase, so that output variables have the same value as the

transfer function of the output variable with respect to the

input.

The generation of white noise by resistors and semiconductor

devices can also be simulated with the ac small-signal portion of

SPICE.  Equivalent noise source values are determined

automatically from the small-signal operating point of the

circuit, and the contribution of each noise source is added at a

given summing point.  The total output noise level and the

equivalent input noise level are determined at each frequency

point.  The output and input noise levels are normalized with

respect to the square root of the noise bandwidth and have the

units VOLTS/RT HZ or AMPS/RT HZ.  The output noise and equivalent

input noise can be printed or plotted in the same fashion as

other output variables.  No additional input data are necessary

for this analysis.

Flicker noise sources can be simulated in the noise analysis

by including values for the parameters KF and AF on the

appropriate device model lines.

3

The distortion characteristics of a circuit in the

small-signal mode can be simulated as a part of the ac

small-signal analysis.  The analysis is performed assuming that

one or two signal frequencies are imposed at the input.

The frequency range and the noise and distortion analysis

parameters are specified on the .AC, .NOISE, and .DISTO control

lines.

1.3.  TRANSIENT ANALYSIS ____  _________ ________

The transient analysis portion of SPICE computes the

transient output variables as a function of time over a

user-specified time interval.  The initial conditions are

automatically determined by a dc analysis.  All sources which are

not time dependent (for example, power supplies) are set to their

dc value.  For large-signal sinusoidal simulations, a Fourier

analysis of the output waveform can be specified to obtain the

frequency domain Fourier coefficients.  The transient time

interval and the Fourier analysis options are specified on the

.TRAN and .FOURIER control lines.

1.4.  ANALYSIS AT DIFFERENT TEMPERATURES ____  ________ __ _________ ____________

All input data for SPICE is assumed to have been measured at

27 deg C (300 deg K).  The simulation also assumes a nominal

temperature of 27 deg C.  The circuit can be simulated at other

temperatures by using a .TEMP control line.

Temperature appears explicitly in the exponential terms of

the bjt and diode model equations.  In addition, saturation

currents have a built-in temperature dependence.  The temperature

dependence of the saturation current in the bjt models is

determined by:

IS(T1) = IS(T0)*((T1/T0)**XTI)*EXP(Q*EG*(T1-T0)/(K*T1*T0))

where K is Boltzmann's constant, Q is the electronic charge, EG

is the energy gap which is a model parameter, and XTI is the

saturation current temperature exponent (also a model parameter,

and usually equal to 3).  The temperature dependence of forward

and reverse BETA is according to the formula:

BETA(T1)=BETA(T0)*(T1/T0)**XTB

where T1 and T0 are in degrees Kelvin, and XTB is a user-supplied

model parameter.  Temperature effects on BETA are carried out by

appropriate adjustment to the values of BF, ISE, BR, and ISC.

Temperature dependence of the saturation current in the junction

diode model is determined by:

IS(T1) = IS(T0)*((T1/T0)**(XTI/N))*EXP(Q*EG*(T1-T0)/(K*N*T1*T0))

4

where N is the emission coefficient, which is a model parameter,

and the other symbols have the same meaning as above.  Note that

for Schottky barrier diodes, the value of the saturation current

temperature exponent, XTI, is usually 2.

Temperature appears explicitly in the value of junction

potential, PHI, for all the device models.  The temperature

dependence is determined by:

PHI(TEMP) = K*TEMP/Q*LOG(NA*ND/NI(TEMP)**2)

where K is Boltzmann's constant, Q is the electronic charge, NA

is the acceptor impurity density, ND is the donor impurity

density, NI is the intrinsic concentration, and EG is the energy

gap.

Temperature appears explicitly in the value of surface

mobility, UO, for the mosfet model.  The temperature dependence

is determined by:

UO(TEMP) = UO(TNOM)/(TEMP/TNOM)**(1.5)

The effects of temperature on resistors is modeled by the

formula:

VALUE(TEMP) = VALUE(TNOM)*(1+TC1*(TEMP-TNOM)+TC2*(TEMP-TNOM)**2))

where TEMP is the circuit temperature, TNOM is the nominal

temperature, and TC1 and TC2 are the first and second order

temperature coefficients.

2.  CONVERGENCE __  ___________

Both dc and transient solutions are obtained by an iterative

process which is terminated when both of the following conditions

hold:

1.  The nonlinear branch currents converge to within a

tolerance of 0.1 percent or 1 picoamp (1.0E-12 amp),

whichever is larger.

2.  The node voltages converge to within a tolerance of 0.1

percent or 1 microvolt (1.0E-6 volt), whichever is

larger.

Although the algorithm used in SPICE has been found to be

very reliable, in some cases it will fail to converge to a

solution.  When this failure occurs, the program will print the

node voltages at the last iteration and terminate the job.  In

such cases, the node voltages that are printed are not

necessarily correct or even close to the correct solution.

5

Failure to converge in the dc analysis is usually due to an

error in specifying circuit connections, element values, or model

parameter values.  Regenerative switching circuits or circuits

with positive feedback probably will not converge in the dc

analysis unless the OFF option is used for some of the devices in

the feedback path,

or the .NODESET line is used to force the circuit to

converge to the desired state.

3.  INPUT FORMAT __  _____ ______

The input format for SPICE is of the free format type.

Fields on a line are separated by one or more blanks, a comma, an

equal (=) sign, or a left or right parenthesis; extra spaces are

ignored.  A line may be continued by entering a plus sign (+) in

column 1 of the following line; SPICE continues reading beginning

with column 2.

A name field must begin with a letter (A through Z) and

cannot contain any delimiters.  Only the first eight characters

of the name are used.

A number field may be an integer field (12, -44), a floating

point field (3.14159), either an integer or floating point number

followed by an integer exponent (1E-14, 2.65E3), or either an

integer or a floating point number followed by one of the

following scale factors:

T=1E12   G=1E9    MEG=1E6   K=1E3     MIL=25.4E-6

M=1E-3   U=1E-6   N=1E-9    P=1E-12   F=1E-15

Letters immediately following a number that are not scale factors

are ignored, and letters immediately following a scale factor are

ignored.  Hence, 10, 10V, 10VOLTS, and 10HZ all represent the

same number, and M, MA, MSEC, and MMHOS all represent the same

scale factor.  Note that 1000, 1000.0, 1000HZ, 1E3, 1.0E3, 1KHZ,

and 1K all represent the same number.

4.  CIRCUIT DESCRIPTION __  _______ ___________

The circuit to be analyzed is described to SPICE by a set of

element lines, which define the circuit topology and element

values, and a set of control lines, which define the model

parameters and the run controls.  The first line in the input

file must be a TITLE line, and the last line must be a .END line.

The order of the remaining lines is arbitrary (except, of course,

that continuation lines must immediately follow the line being

continued).

6

Each element in the circuit is specified by an element line

that contains the element name, the circuit nodes to which the

element is connected, and the values of the parameters that

determine the electrical characteristics of the element.  The

first letter of the element name specifies the element type.  The

format for the SPICE element types is given in what follows.  The

strings XXXXXXX, YYYYYYY, and ZZZZZZZ denote arbitrary

alphanumeric strings.  For example, a resistor name must begin

with the letter R and can contain from one to eight characters.

Hence, R, R1, RSE, ROUT, and R3AC2ZY are valid resistor names.

Data fields that are enclosed in lt and gt signs '< >' are

optional.  All indicated punctuation (parentheses, equal signs,

etc.) are required.  With respect to branch voltages and

currents, SPICE uniformly uses the associated reference

convention (current flows in the direction of voltage drop).

Nodes must be nonnegative integers but need not be numbered

sequentially.  The datum (ground) node must be numbered zero.

The circuit cannot contain a loop of voltage sources and/or

inductors and cannot contain a cutset of current sources and/or

capacitors.  Each node in the circuit must have a dc path to

ground.  Every node must have at least two connections except for

transmission line nodes (to permit unterminated transmission

lines) and mosfet substrate nodes (which have two internal

connections anyway).

5.  TITLE LINE COMMENT LINES AND END LINE __  _____ ____ _______ _____ ___ ___ ____

5.1.  .TITLE LINE ____  ______ ____

Examples: _________

POWER AMPLIFIER CIRCUIT

TEST OF CAM CELL

This line must be the first line in the input file.  Its

contents are printed verbatim as the heading for each section of

output.

5.2.  .END LINE ____  ____ ____

Examples: _________

.END

This line must always be the last line in the input file.

Note that the period is an integral part of the name.

7

5.3.  COMMENT LINE ____  _______ ____

General Form: _______ _____

* <ANY COMMENT>

Examples: _________

* RF=1K      GAIN SHOULD BE 100

* MAY THE FORCE BE WITH MY CIRCUIT

The asterisk in the first column indicates that this line is

a comment line.  Comment lines may be placed anywhere in the

circuit description.

6.  ELEMENT LINES __  _______ _____

6.1.  RESISTORS ____  _________

General Form: _______ _____

RXXXXXXX N1 N2 VALUE <TC=TC1<,TC2>>

Examples: _________

R1 1 2 100

RC1 12 17 1K TC=0.001,0.015

N1 and N2 are the two element nodes.  VALUE is the

resistance (in ohms) and may be positive or negative but not

zero.  TC1 and TC2 are the (optional) temperature coefficients;

if not specified, zero is assumed for both.  The value of the

resistor as a function of temperature is given by:

VALUE(TEMP) = VALUE(TNOM)*(1+TC1*(TEMP-TNOM)+TC2*(TEMP-TNOM)**2))

6.2.  CAPACITORS AND INDUCTORS ____  __________ ___ _________

General Form: _______ _____

CXXXXXXX N+ N- VALUE <IC=INCOND>

LYYYYYYY N+ N- VALUE <IC=INCOND>

Examples: _________

CBYP 13 0 1UF

COSC 17 23 10U IC=3V

LLINK 42 69 1UH

LSHUNT 23 51 10U IC=15.7MA

8

N+ and N- are the positive and negative element nodes,

respectively.  VALUE is the capacitance in farads or the

inductance in henries.

For the capacitor, the (optional) initial condition is the

initial (time-zero) value of capacitor voltage (in volts).  For

the inductor, the (optional) initial condition is the initial

(time-zero) value of inductor current (in amps) that flows from

N+, through the inductor, to N-.  Note that the initial

conditions (if any) apply 'only' if the UIC option is specified

on the .TRAN line.

Nonlinear capacitors and inductors can be described.

General Form: _______ _____

CXXXXXXX N+ N- POLY C0 C1 C2 ... <IC=INCOND>

LYYYYYYY N+ N- POLY L0 L1 L2 ... <IC=INCOND>

C0 C1 C2 ...(and L0 L1 L2 ...) are the coefficients of a

polynomial describing the element value.  The capacitance is

expressed as a function of the voltage across the element while

the inductance is a function of the current through the inductor.

The value is computed as

VALUE=C0+C1*V+C2*V**2+...

VALUE=L0+L1*I+L2*I**2+...

where V is the voltage across the capacitor and I the current

flowing in the inductor.

6.3.  COUPLED MUTUAL INDUCTORS ____  _______ ______ _________

General Form: _______ _____

KXXXXXXX LYYYYYYY LZZZZZZZ VALUE

Examples: _________

K43 LAA LBB 0.999

KXFRMR L1 L2 0.87

LYYYYYYY and LZZZZZZZ are the names of the two coupled

inductors, and VALUE is the coefficient of coupling, K, which

must be greater than 0 and less than or equal to 1.  Using the

'dot' convention, place a 'dot' on the first node of each

inductor.

9

6.4.  TRANSMISSION LINES (LOSSLESS) ____  ____________ _____ __________

General Form: _______ _____

TXXXXXXX N1 N2 N3 N4 Z0=VALUE <TD=VALUE> <F=FREQ <NL=NRMLEN>>

+                    <IC=V1,I1,V2,I2>

Examples: _________

T1 1 0 2 0 Z0=50 TD=10NS

N1 and N2 are the nodes at port 1; N3 and N4 are the nodes

at port 2.  Z0 is the characteristic impedance.  The length of

the line may be expressed in either of two forms.  The

transmission delay, TD, may be specified directly (as TD=10NS,

for example).  Alternatively, a frequency F may be given,

together with NL, the normalized electrical length of the

transmission line with respect to the wavelength in the line at

the frequency F.  If a frequency is specified but NL is omitted,

0.25 is assumed (that is, the frequency is assumed to be the

quarter-wave frequency).  Note that although both forms for

expressing the line length are indicated as optional, one of the

two must be specified.

Note that this element models only one propagating mode.  If

all four nodes are distinct in the actual circuit, then two modes

may be excited.  To simulate such a situation, two

transmission-line elements are required.  (See the example in

appendix A for further clarification.)

The (optional) initial condition specification consists of

the voltage and current at each of the transmission line ports.

Note that the initial conditions (if any) apply 'only' if the UIC

option is specified on the .TRAN line.

One should be aware that SPICE will use a transient

time-step which does not exceed 1/2 the minimum transmission line

delay.  Therefore very short transmission lines (compared with

the analysis time frame) will cause long run times.

6.5.  LINEAR DEPENDENT SOURCES ____  ______ _________ _______

SPICE allows circuits to contain linear dependent sources

characterized by any of the four equations

I=G*V          V=E*V          I=F*I          V=H*I

where G, E, F, and H are constants representing transconductance,

voltage gain, current gain, and transresistance, respectively.

Note: A more complete description of dependent sources as _____

implemented in SPICE is given in appendix B.

10

6.6.  LINEAR VOLTAGE-CONTROLLED CURRENT SOURCES ____  ______ __________________ _______ _______

General Form: _______ _____

GXXXXXXX N+ N- NC+ NC- VALUE

Examples: _________

G1 2 0 5 0 0.1MMHO

N+ and N- are the positive and negative nodes, respectively.

Current flow is from the positive node, through the source, to

the negative node.  NC+ and NC- are the positive and negative

controlling nodes, respectively.  VALUE is the transconductance

(in mhos).

6.7.  LINEAR VOLTAGE-CONTROLLED VOLTAGE SOURCES ____  ______ __________________ _______ _______

General Form: _______ _____

EXXXXXXX N+ N- NC+ NC- VALUE

Examples: _________

E1 2 3 14 1 2.0

N+ is the positive node, and N- is the negative node.  NC+

and NC- are the positive and negative controlling nodes,

respectively.  VALUE is the voltage gain.

6.8.  LINEAR CURRENT-CONTROLLED CURRENT SOURCES ____  ______ __________________ _______ _______

General Form: _______ _____

FXXXXXXX N+ N- VNAM VALUE

Examples: _________

F1 13 5 VSENS 5

N+ and N- are the positive and negative nodes, respectively.

Current flow is from the positive node, through the source, to

the negative node.  VNAM is the name of a voltage source through

which the controlling current flows.  The direction of positive

controlling current flow is from the positive node, through the

source, to the negative node of VNAM.  VALUE is the current gain.

6.9.  LINEAR CURRENT-CONTROLLED VOLTAGE SOURCES ____  ______ __________________ _______ _______

General Form: _______ _____

HXXXXXXX N+ N- VNAM VALUE

11

Examples: _________

HX 5 17 VZ 0.5K

N+ and N- are the positive and negative nodes, respectively.

VNAM is the name of a voltage source through which the

controlling current flows.  The direction of positive controlling

current flow is from the positive node, through the source, to

the negative node of VNAM.  VALUE is the transresistance (in

ohms).

6.10.  INDEPENDENT SOURCES _____  ___________ _______

General Form: _______ _____

VXXXXXXX N+ N- <<DC> DC/TRAN VALUE> <AC <ACMAG <ACPHASE>>>

IYYYYYYY N+ N- <<DC> DC/TRAN VALUE> <AC <ACMAG <ACPHASE>>>

Examples: _________

VCC 10 0 DC 6

VIN 13 2 0.001 AC 1 SIN(0 1 1MEG)

ISRC 23 21 AC 0.333 45.0 SFFM(0 1 10K 5 1K)

VMEAS 12 9

N+ and N- are the positive and negative nodes, respectively.

Note that voltage sources need not be grounded.  Positive current

is assumed to flow from the positive node, through the source, to

the negative node.  A current source of positive value, will

force current to flow out of the N+ node, through the source, and

into the N- node.  Voltage sources, in addition to being used for

circuit excitation, are the 'ammeters' for SPICE, that is, zero

valued voltage sources may be inserted into the circuit for the

purpose of measuring current.  They will, of course, have no

effect on circuit operation since they represent short-circuits.

DC/TRAN is the dc and transient analysis value of the

source.  If the source value is zero both for dc and transient

analyses, this value may be omitted.  If the source value is

time-invariant (e.g., a power supply), then the value may

optionally be preceded by the letters DC.

ACMAG is the ac magnitude and ACPHASE is the ac phase.  The

source is set to this value in the ac analysis.  If ACMAG is

omitted following the keyword AC, a value of unity is assumed.

if ACPHASE is omitted, a value of zero is assumed.  If the source

is not an ac small-signal input, the keyword AC and the ac values

are omitted.

Any independent source can be assigned a time-dependent

value for transient analysis.  If a source is assigned a

12

time-dependent value, the time-zero value is used for dc

analysis.  There are five independent source functions: pulse,

exponential, sinusoidal, piece-wise linear, and single-frequency

fm.  If parameters other than source values are omitted or set to

zero, the default values shown will be assumed.  (TSTEP is the

printing increment and TSTOP is the final time (see the .TRAN

line for explanation)).

6.11.  PULSE      PULSE(V1 V2 TD TR TF PW PER) _____  _____

Examples: _________

VIN 3 0 PULSE(-1 1 2NS 2NS 2NS 50NS 100NS)

PARAMETERS              DEFAULT VALUES         UNITS

V1 (INITIAL VALUE)                       VOLTS OR AMPS

V2 (PULSED VALUE)                        VOLTS OR AMPS

TD (DELAY TIME)         0.0              SECONDS

TR (RISE TIME)          TSTEP            SECONDS

TF (FALL TIME)          TSTEP            SECONDS

PW (PULSE WIDTH)        TSTOP            SECONDS

PER(PERIOD)             TSTOP            SECONDS

A single pulse so specified is described by the following

table:

TIME          VALUE

0             V1

TD            V1

TD+TR         V2

TD+TR+PW      V2

TD+TR+PW+TF   V1

TSTOP         V1

Intermediate points are determined by linear interpolation.

6.12.  SINUSOIDAL      SIN(VO VA FREQ TD THETA) _____  __________

Examples: _________

VIN 3 0 SIN(0 1 100MEG 1NS 1E10)

PARAMETERS                DEFAULT VALUE   UNITS

VO     (OFFSET)                           VOLTS OR AMPS

VA     (AMPLITUDE)                        VOLTS OR AMPS

FREQ   (FREQUENCY)        1/TSTOP         HZ

TD     (DELAY)            0.0             SECONDS

THETA  (DAMPING FACTOR)   0.0             1/SECONDS

13

The shape of the waveform is described by the following table:

TIME       VALUE

0 - TD     VO

TD - TSTOP VO + VA*EXP(-(TIME-TD)*THETA)*SINE(TWOPI*FREQ*(TIME+TD))

6.13.  EXPONENTIAL      EXP(V1 V2 TD1 TAU1 TD2 TAU2) _____  ___________

Examples: _________

VIN 3 0 EXP(-4 -1 2NS 30NS 60NS 40NS)

PARAMETERS                  DEFAULT VALUES   UNITS

V1   (INITIAL VALUE)                         VOLTS OR AMPS

V2   (PULSED VALUE)                          VOLTS OR AMPS

TD1  (RISE DELAY TIME)      0.0              SECONDS

TAU1 (RISE TIME CONSTANT)   TSTEP            SECONDS

TD2  (FALL DELAY TIME)      TD1+TSTEP        SECONDS

TAU2 (FALL TIME CONSTANT)   TSTEP            SECONDS

The shape of the waveform is described by the following table:

TIME           VALUE

0 TO TD1       V1

TD1 TO TD2     V1+(V2-V1)*(1-EXP(-(TIME-TD1)/TAU1))

TD2 TO TSTOP   V1+(V2-V1)*(1-EXP(-(TIME-TD1)/TAU1))

+(V1-V2)*(1-EXP(-(TIME-TD2)/TAU2))

6.14.  PIECE-WISE LINEAR      PWL(T1 V1 <T2 V2 T3 V3 T4 V4 ...>) _____  __________ ______

Examples: _________

VCLOCK 7 5 PWL(0 -7 10NS -7 11NS -3 17NS -3 18NS -7 50NS -7)

PARAMETERS AND DEFAULT VALUES

Each pair of values (Ti, Vi) specifies that the value of the

source is Vi (in volts or amps) at time = Ti.  The value of the

source at intermediate values of time is determined by using

linear interpolation on the input values.

6.15.  SINGLE-FREQUENCY FM      SFFM(VO VA FC MDI FS) _____  ________________ __

Examples: _________

V1 12 0 SFFM(0 1M 20K 5 1K)

14

PARAMETERS                DEFAULT VALUES   UNITS

VO  (OFFSET)                               VOLTS OR AMPS

VA  (AMPLITUDE)                            VOLTS OR AMPS

FC  (CARRIER FREQUENCY)   1/TSTOP          HZ

MDI (MODULATION INDEX)

FS  (SIGNAL FREQUENCY)    1/TSTOP          HZ

The shape of the waveform is described by the following equation:

VALUE = VO + VA*SINE((TWOPI*FC*TIME) + MDI*SINE(TWOPI*FS*TIME))

7.  SEMICONDUCTOR DEVICES __  _____________ _______

The elements that have been described to this point

typically require only a few parameter values to specify

completely the electrical characteristics of the element.

However, the models for the four semiconductor devices that are

included in the SPICE program require many parameter values.

Moreover, many devices in a circuit often are defined by the same

set of device model parameters.  For these reasons, a set of

device model parameters is defined on a separate .MODEL line and

assigned a unique model name.  The device element lines in SPICE

then reference the model name.  This scheme alleviates the need

to specify all of the model parameters on each device element

line.

Each device element line contains the device name, the nodes

to which the device is connected, and the device model name.  In

addition, other optional parameters may be specified for each

device: geometric factors and an initial condition.

The area factor used on the diode, bjt and jfet device line

determines the number of equivalent parallel devices of a

specified model.  The affected parameters are marked with an

asterisk under the heading 'AREA' in the model descriptions

below.  Several geometric factors associated with the channel and

the drain and source diffusions can be specified on the mosfet

device line.

Two different forms of initial conditions may be specified

for devices.  The first form is included to improve the dc

convergence for circuits that contain more than one stable state.

If a device is specified OFF, the dc operating point is

determined with the terminal voltages for that device set to

zero.  After convergence is obtained, the program continues to

iterate to obtain the exact value for the terminal voltages.  If

a circuit has more than one dc stable state, the OFF option can

be used to force the solution to correspond to a desired state.

If a device is specified OFF when in reality the device is

15

conducting, the program will still obtain the correct solution

(assuming the solutions converge) but more iterations will be

required since the program must independently converge to two

separate solutions.  The .NODESET line serves a similar purpose

as the OFF option.  The .NODESET option is easier to apply and is

the preferred means to aid convergence.

The second form of initial conditions are specified for use

with the transient analysis.  These are true 'INITIAL CONDITIONS'

as opposed to the convergence aids above.  See the description of

the .IC line and the .TRAN line for a detailed explanation of

initial conditions.

7.1.  JUNCTION DIODES ____  ________ ______

General Form: _______ _____

DXXXXXXX N+ N- MNAME <AREA> <OFF> <IC=VD>

Examples: _________

DBRIDGE 2 10 DIODE1

DCLMP 3 7 DMOD 3.0 IC=0.2

N+ and N- are the positive and negative nodes, respectively.

MNAME is the model name, AREA is the area factor, and OFF

indicates an (optional) starting condition on the device for dc

analysis.  If the area factor is omitted, a value of 1.0 is

assumed.  The (optional) initial condition specification using

IC=VD is intended for use with the UIC option on the .TRAN line,

when a transient analysis is desired starting from other than the

quiescent operating point.

7.2.  BIPOLAR JUNCTION TRANSISTORS (BJT'S) ____  _______ ________ ___________ _______

General Form: _______ _____

QXXXXXXX NC NB NE <NS> MNAME <AREA> <OFF> <IC=VBE,VCE>

Examples: _________

Q23 10 24 13 QMOD IC=0.6,5.0

Q50A 11 26 4 20 MOD1

NC, NB, and NE are the collector, base, and emitter nodes,

respectively.  NS is the (optional) substrate node.  If

unspecified, ground is used.  MNAME is the model name, AREA is

the area factor, and OFF indicates an (optional) initial

condition on the device for the dc analysis.  If the area factor

is omitted, a value of 1.0 is assumed.  The (optional) initial

condition specification using IC=VBE,VCE is intended for use with

16

the UIC option on the .TRAN line, when a transient analysis is

desired starting from other than the quiescent operating point.

See the .IC line description for a better way to set transient

initial conditions.

7.3.  JUNCTION FIELD-EFFECT TRANSISTORS (JFET'S) ____  ________ ____________ ___________ ________

General Form: _______ _____

JXXXXXXX ND NG NS MNAME <AREA> <OFF> <IC=VDS,VGS>

Examples: _________

J1 7 2 3 JM1 OFF

ND, NG, and NS are the drain, gate, and source nodes,

respectively.  MNAME is the model name, AREA is the area factor,

and OFF indicates an (optional) initial condition on the device

for dc analysis.  If the area factor is omitted, a value of 1.0

is assumed.  The (optional) initial condition specification,

using IC=VDS,VGS is intended for use with the UIC option on the

.TRAN line, when a transient analysis is desired starting from

other than the quiescent operating point.  See the .IC line for a

better way to set initial conditions.

7.4.  MOSFET'S ____  ________

General Form: _______ _____

MXXXXXXX ND NG NS NB MNAME <L=VAL> <W=VAL> <AD=VAL> <AS=VAL>

+ <PD=VAL> <PS=VAL> <NRD=VAL> <NRS=VAL> <OFF> <IC=VDS,VGS,VBS>

Examples: _________

M1 24 2 0 20 TYPE1

M31 2 17 6 10 MODM L=5U W=2U

M31 2 16 6 10 MODM 5U 2U

M1 2 9 3 0 MOD1 L=10U W=5U AD=100P AS=100P PD=40U PS=40U

M1 2 9 3 0 MOD1 10U 5U 2P 2P

ND, NG, NS, and NB are the drain, gate, source, and bulk

(substrate) nodes, respectively.  MNAME is the model name.  L and

W are the channel length and width, in meters.  AD and AS are the

areas of the drain and source diffusions, in sq-meters.  Note

that the suffix U specifies microns (1E-6 m) and P sq-microns

(1E-12 sq-m).  If any of L, W, AD, or AS are not specified,

default values are used.  The user may specify the values to be

used for these default parameters on the .OPTIONS line.  The use

of defaults simplifies input file preparation, as well as the

editing required if device geometries are to be changed.  PD and

PS are the perimeters of the drain and source junctions, in

17

meters.  NRD and NRS designate the equivalent number of squares

of the drain and source diffusions; these values multiply the

sheet resistance RSH specified on the .MODEL line for an accurate

representation of the parasitic series drain and source

resistance of each transistor.  PD and PS default to 0.0 while

NRD and NRS to 1.0.  OFF indicates an (optional) initial

condition on the device for dc analysis.  The (optional) initial

condition specification using IC=VDS,VGS,VBS is intended for use

with the UIC option on the .TRAN line, when a transient analysis

is desired starting from other than the quiescent operating

point.  See the .IC line for a better and more convenient way to

specify transient initial conditions.

7.5.  .MODEL LINE ____  ______ ____

General Form: _______ _____

.MODEL MNAME TYPE(PNAME1=PVAL1 PNAME2=PVAL2 ... )

Examples: _________

.MODEL MOD1 NPN BF=50 IS=1E-13 VBF=50

The .MODEL line specifies a set of model parameters that

will be used by one or more devices.  MNAME is the model name,

and TYPE is one of the following seven types:

NPN    NPN BJT MODEL

PNP    PNP BJT MODEL

D      DIODE MODEL

NJF    N-CHANNEL JFET MODEL

PJF    P-CHANNEL JFET MODEL

NMOS   N-CHANNEL MOSFET MODEL

PMOS   P-CHANNEL MOSFET MODEL

Parameter values are defined by appending the parameter

name, as given below for each model type, followed by an equal

sign and the parameter value.  Model parameters that are not

given a value are assigned the default values given below for

each model type.

7.6.  DIODE MODEL ____  _____ _____

The dc characteristics of the diode are determined by the

parameters IS and N.  An ohmic resistance, RS, is included.

Charge storage effects are modeled by a transit time, TT, and a

nonlinear depletion layer capacitance which is determined by the

parameters CJO, VJ, and M.  The temperature dependence of the

saturation current is defined by the parameters EG, the energy

gap and XTI, the saturation current temperature exponent.

Reverse breakdown is modeled by an exponential increase in the

18

reverse diode current and is determined by the parameters BV and

IBV (both of which are positive numbers).

NAME PARAMETER                     UNITS DEFAULT  EXAMPLE AREA

IS  SATURATION CURRENT             A    1.0E-14  1.0E-14  *

RS  OHMIC RESISTANCE               OHM  0        10       *

N   EMISSION COEFFICIENT           -    1        1.0

TT  TRANSIT-TIME                   SEC  0        0.1NS

CJO ZERO-BIAS JUNCTION CAPACITANCE F    0        2PF      *

VJ  JUNCTION POTENTIAL             V    1        0.6

M   GRADING COEFFICIENT            -    0.5      0.5

EG  ACTIVATION ENERGY              EV   1.11     1.11 SI

0.69 SBD

0.67 GE

XTI SATURATION-CURRENT TEMP. EXP   -    3.0      3.0 JN

2.0 SBD

KF  FLICKER NOISE COEFFICIENT      -    0

AF  FLICKER NOISE EXPONENT         -    1

FC  COEFFICIENT FOR FORWARD-BIAS   -    0.5

DEPLETION CAPACITANCE FORMULA

BV  REVERSE BREAKDOWN VOLTAGE      V    INFINITE 40.0

IBV CURRENT AT BREAKDOWN VOLTAGE   A    1.0E-3

7.7.  BJT MODELS (BOTH NPN AND PNP) ____  ___ ______ _____ ___ ___ ____

The bipolar junction transistor model in SPICE is an

adaptation of the integral charge control model of Gummel and

Poon.  This modified Gummel-Poon model extends the original model

to include several effects at high bias levels.  The model will

automatically simplify to the simpler Ebers-Moll model when

certain parameters are not specified.  The parameter names used

in the modified Gummel-Poon model have been chosen to be more

easily understood by the program user, and to reflect better both

physical and circuit design thinking.

The dc model is defined by the parameters IS, BF, NF, ISE,

IKF, and NE which determine the forward current gain

characteristics, IS, BR, NR, ISC, IKR, and NC which determine the

reverse current gain characteristics, and VAF and VAR which

determine the output conductance for forward and reverse regions.

Three ohmic resistances RB, RC, and RE are included, where RB can

be high current dependent.  Base charge storage is modeled by

forward and reverse transit times, TF and TR, the forward transit

time TF being bias dependent if desired, and nonlinear depletion

layer capacitances which are determined by CJE, VJE, and MJE for

the B-E junction , CJC, VJC, and MJC for the B-C junction and

CJS, VJS, and MJS for the C-S (collector-substrate) junction.

The temperature dependence of the saturation current, IS, is

determined by the energy-gap, EG, and the saturation current

temperature exponent, XTI.  Additionally base current temperature

19

dependence is modeled by the beta temperature exponent XTB in the

new model.

The bjt parameters used in the modified Gummel-Poon model

are listed below.  The parameter names used in earlier versions

of SPICE2 are still accepted.

MODIFIED GUMMEL-POON BJT PARAMETERS.

NAME PARAMETER                            UNITS DEFAULT  EXAMPLE AR

EA

IS   TRANSPORT SATURATION CURRENT          A    1.0E-16  1.0E-15 *

BF   IDEAL MAXIMUM FORWARD BETA            -    100      100

NF   FORWARD CURRENT EMISSION COEFFICIENT  -    1.0      1

VAF  FORWARD EARLY VOLTAGE                 V    INFINITE 200

IKF  CORNER FOR FORWARD BETA

HIGH CURRENT ROLL-OFF                 A    INFINITE 0.01    *

ISE  B-E LEAKAGE SATURATION CURRENT        A    0        1.0E-13 *

NE   B-E LEAKAGE EMISSION COEFFICIENT      -    1.5      2

BR   IDEAL MAXIMUM REVERSE BETA            -    1        0.1

NR   REVERSE CURRENT EMISSION COEFFICIENT  -    1        1

VAR  REVERSE EARLY VOLTAGE                 V    INFINITE 200

IKR  CORNER FOR REVERSE BETA

HIGH CURRENT ROLL-OFF                 A    INFINITE 0.01    *

ISC  B-C LEAKAGE SATURATION CURRENT        A    0        1.0E-13 *

NC   B-C LEAKAGE EMISSION COEFFICIENT      -    2        1.5

RB   ZERO BIAS BASE RESISTANCE             OHMS 0        100     *

IRB  CURRENT WHERE BASE RESISTANCE

FALLS HALFWAY TO ITS MIN VALUE        A    INFINITE 0.1     *

RBM  MINIMUM BASE RESISTANCE

AT HIGH CURRENTS                      OHMS RB       10      *

RE   EMITTER RESISTANCE                    OHMS 0        1       *

RC   COLLECTOR RESISTANCE                  OHMS 0        10      *

CJE  B-E ZERO-BIAS DEPLETION CAPACITANCE   F    0        2PF     *

VJE  B-E BUILT-IN POTENTIAL                V    0.75     0.6

MJE  B-E JUNCTION EXPONENTIAL FACTOR       -    0.33     0.33

TF   IDEAL FORWARD TRANSIT TIME            SEC  0        0.1NS

XTF  COEFFICIENT FOR BIAS DEPENDENCE OF TF -    0

VTF  VOLTAGE DESCRIBING VBC

DEPENDENCE OF TF                      V    INFINITE

ITF  HIGH-CURRENT PARAMETER

FOR EFFECT ON TF                      A    0                *

PTF  EXCESS PHASE AT FREQ=1.0/(TF*2PI) HZ  DEG  0

CJC  B-C ZERO-BIAS DEPLETION CAPACITANCE   F    0        2PF     *

VJC  B-C BUILT-IN POTENTIAL                V    0.75     0.5

MJC  B-C JUNCTION EXPONENTIAL FACTOR       -    0.33     0.5

XCJC FRACTION OF B-C DEPLETION CAPACITANCE -    1

CONNECTED TO INTERNAL BASE NODE

TR   IDEAL REVERSE TRANSIT TIME            SEC  0        10NS

CJS  ZERO-BIAS COLLECTOR-SUBSTRATE

CAPACITANCE                           F    0        2PF     *

VJS  SUBSTRATE JUNCTION BUILT-IN POTENTIAL V    0.75

20

MJS  SUBSTRATE JUNCTION EXPONENTIAL FACTOR -    0        0.5

XTB  FORWARD AND REVERSE BETA

TEMPERATURE EXPONENT                  -    0

EG   ENERGY GAP FOR TEMPERATURE

EFFECT ON IS                          EV   1.11

XTI  TEMPERATURE EXPONENT FOR EFFECT ON IS -    3

KF   FLICKER-NOISE COEFFICIENT             -    0

AF   FLICKER-NOISE EXPONENT                -    1

FC   COEFFICIENT FOR FORWARD-BIAS

DEPLETION CAPACITANCE FORMULA         -    0.5

7.8.  JFET MODELS (BOTH N AND P CHANNEL) ____  ____ ______ _____ _ ___ _ ________

The jfet model is derived from the fet model of Shichman and

Hodges.  The dc characteristics are defined by the parameters VTO

and BETA, which determine the variation of drain current with

gate voltage, LAMBDA, which determines the output conductance,

and IS, the saturation current of the two gate junctions.  Two

ohmic resistances, RD and RS, are included.  Charge storage is

modeled by nonlinear depletion layer capacitances for both gate

junctions which vary as the -1/2 power of junction voltage and

are defined by the parameters CGS, CGD, and PB.

NAME   PARAMETER                          UNITS  DEFAULT EXAMPLE AR

EA

VTO    THRESHOLD VOLTAGE                  V      -2.0    -2.0

BETA   TRANSCONDUCTANCE PARAMETER         A/V**2 1.0E-4  1.0E-3  *

LAMBDA CHANNEL LENGTH MODULATION

PARAMETER                          1/V    0       1.0E-4

RD     DRAIN OHMIC RESISTANCE             OHM    0       100     *

RS     SOURCE OHMIC RESISTANCE            OHM    0       100     *

CGS    ZERO-BIAS G-S JUNCTION CAPACITANCE F      0       5PF     *

CGD    ZERO-BIAS G-D JUNCTION CAPACITANCE F      0       1PF     *

PB     GATE JUNCTION POTENTIAL            V      1       0.6

IS     GATE JUNCTION SATURATION CURRENT   A      1.0E-14 1.0E-14 *

KF     FLICKER NOISE COEFFICIENT          -      0

AF     FLICKER NOISE EXPONENT             -      1

FC     COEFFICIENT FOR FORWARD-BIAS       -      0.5

DEPLETION CAPACITANCE FORMULA

7.9.  MOSFET MODELS (BOTH N AND P CHANNEL) ____  ______ ______ _____ _ ___ _ ________

SPICE provides three mosfet device models which differ in

the formulation of the I-V characteristic.  The variable level

specifies the model to be used:

LEVEL=1 ->    SHICHMAN-HODGES

LEVEL=2 ->    MOS2 (AS DESCRIBED IN [1])

LEVEL=3 ->    MOS3, A SEMI-EMPIRICAL MODEL(SEE [1])

21

The dc characteristics of the mosfet are defined by the

device parameters VTO, KP, LAMBDA, PHI and GAMMA.  These

parameters are computed by SPICE if process parameters (NSUB,

TOX, ...) are given, but user-specified values always override.

VTO is positive (negative) for enhancement mode and negative

(positive) for depletion mode N-channel (P-channel) devices.

Charge storage is modeled by three constant capacitors, CGSO,

CGDO, and CGBO which represent overlap capacitances, by the

nonlinear thin-oxide capacitance which is distributed among the

gate, source, drain, and bulk regions, and by the nonlinear

depletion-layer capacitances for both substrate junctions divided

into bottom and periphery, which vary as the MJ and MJSW power of

junction voltage respectively, and are determined by the

parameters CBD, CBS, CJ, CJSW, MJ, MJSW and PB.  There are two

built-in models of the charge storage effects associated with the

thin-oxide.  The default is the piecewise linear

voltage-dependent capacitance model proposed by Meyer.  The

second choice is the charge-controlled capacitance model of Ward

and Dutton [1].  The XQC model parameter acts as a flag and a

coefficient at the same time.  As the former it causes the

program to use Meyer's model whenever larger than 0.5 or not

specified, and the charge-controlled model when between 0 and

0.5.  In the latter case its value defines the share of the

channel charge associated with the drain terminal in the

saturation region.  The thin-oxide charge storage effects are

treated slightly different for the level=1 model.  These

voltage-dependent capacitances are included only if TOX is

specified in the input description and they are represented using

Meyer's formulation.

There is some overlap among the parameters describing the

junctions, e.g., the reverse current can be input either as IS

(in A) or as JS (in A/m**2).  Whereas the first is an absolute

value the second is multiplied by AD and AS to give the reverse

current of the drain and source junctions respectively.  This

methodology has been chosen since there is no sense in relating

always junction characteristics with AD and AS entered on the

device line; the areas can be defaulted.  The same idea applies

also to the zero-bias junction capacitances CBD and CBS (in F) on

one hand, and CJ (in F/m**2) on the other.  The parasitic drain

and source series resistance can be expressed as either RD and RS

(in ohms) or RSH (in ohms/sq.), the latter being multiplied by

the number of squares NRD and NRS input on the device line.

NAME   PARAMETER                           UNITS     DEFAULT EXAMPLE

LEVEL  MODEL INDEX                         -         1

VTO    ZERO-BIAS THRESHOLD VOLTAGE         V         0.0     1.0

KP     TRANSCONDUCTANCE PARAMETER          A/V**2    2.0E-5  3.1E-5

GAMMA  BULK THRESHOLD PARAMETER            V**0.5    0.0     0.37

PHI    SURFACE POTENTIAL                   V         0.6     0.65

LAMBDA CHANNEL-LENGTH MODULATION

22

(MOS1 AND MOS2 ONLY)                1/V       0.0     0.02

RD     DRAIN OHMIC RESISTANCE              OHM       0.0     1.0

RS     SOURCE OHMIC RESISTANCE             OHM       0.0     1.0

CBD    ZERO-BIAS B-D JUNCTION CAPACITANCE  F         0.0     20FF

CBS    ZERO-BIAS B-S JUNCTION CAPACITANCE  F         0.0     20FF

IS     BULK JUNCTION SATURATION CURRENT    A         1.0E-14 1.0E-15

PB     BULK JUNCTION POTENTIAL             V         0.8     0.87

CGSO   GATE-SOURCE OVERLAP CAPACITANCE

PER METER CHANNEL WIDTH             F/M       0.0     4.0E-11

CGDO   GATE-DRAIN OVERLAP CAPACITANCE

PER METER CHANNEL WIDTH             F/M       0.0     4.0E-11

CGBO   GATE-BULK OVERLAP CAPACITANCE

PER METER CHANNEL LENGTH            F/M       0.0     2.0E-10

RSH    DRAIN AND SOURCE DIFFUSION

SHEET RESISITANCE                   OHM/SQ.   0.0     10.0

CJ     ZERO-BIAS BULK JUNCTION BOTTOM CAP.

PER SQ-METER OF JUNCTION AREA       F/M**2    0.0     2.0E-4

MJ     BULK JUNCTION BOTTOM GRADING COEF.  -         0.5     0.5

CJSW   ZERO-BIAS BULK JUNCT. SIDEWALL CAP.

PER METER OF JUNCTION PERIMETER     F/M       0.0     1.0E-9

MJSW   BULK JUNCTION SIDEWALL GRADING COEF.-         0.33

JS     BULK JUNCTION SATURATION CURRENT

PER SQ-METER OF JUNCTION AREA       A/M**2            1.0E-8

TOX    OXIDE THICKNESS                     METER     1.0E-7  1.0E-7

NSUB   SUBSTRATE DOPING                    1/CM**3   0.0     4.0E15

NSS    SURFACE STATE DENSITY               1/CM**2   0.0     1.0E10

NFS    FAST SURFACE STATE DENSITY          1/CM**2   0.0     1.0E10

TPG    TYPE OF GATE MATERIAL:              -         1.0

+1 OPP. TO SUBSTRATE

-1 SAME AS SUBSTRATE

0  AL GATE

XJ     METALLURGICAL JUNCTION DEPTH        METER     0.0     1U

LD     LATERAL DIFFUSION                   METER     0.0     0.8U

UO     SURFACE MOBILITY                    CM**2/V-S 600     700

UCRIT  CRITICAL FIELD FOR MOBILITY

DEGRADATION (MOS2 ONLY)             V/CM      1.0E4   1.0E4

UEXP   CRITICAL FIELD EXPONENT IN

MOBILITY DEGRADATION (MOS2 ONLY)    -         0.0     0.1

UTRA   TRANSVERSE FIELD COEF (MOBILITY)

(MOS2 ONLY)                         -         0.0     0.3

VMAX   MAXIMUM DRIFT VELOCITY OF CARRIERS  M/S       0.0     5.0E4

NEFF   TOTAL CHANNEL CHARGE (FIXED AND

MOBILE) COEFFICIENT (MOS2 ONLY)     -         1.0     5.0

XQC    THIN-OXIDE CAPACITANCE MODEL FLAG

AND COEFFICIENT OF CHANNEL CHARGE

SHARE ATTRIBUTED TO DRAIN (0-0.5)   -         1.0     0.4

KF     FLICKER NOISE COEFFICIENT           -         0.0     1.0E-26

AF     FLICKER NOISE EXPONENT              -         1.0     1.2

FC     COEFFICIENT FOR FORWARD-BIAS

DEPLETION CAPACITANCE FORMULA       -         0.5

DELTA  WIDTH EFFECT ON THRESHOLD VOLTAGE

(MOS2 AND MOS3)                     -         0.0     1.0

23

THETA  MOBILITY MODULATION (MOS3 ONLY)     1/V       0.0     0.1

ETA    STATIC FEEDBACK (MOS3 ONLY)         -         0.0     1.0

KAPPA  SATURATION FIELD FACTOR (MOS3 ONLY) -         0.2     0.5

8.  SUBCIRCUITS __  ___________

A subcircuit that consists of SPICE elements can be defined

and referenced in a fashion similar to device models.  The

subcircuit is defined in the input file by a grouping of element

lines; the program then automatically inserts the group of

elements wherever the subcircuit is referenced.  There is no

limit on the size or complexity of subcircuits, and subcircuits

may contain other subcircuits.  An example of subcircuit usage is

given in appendix A.

24

8.1.  .SUBCKT LINE ____  _______ ____

General Form: _______ _____

.SUBCKT SUBNAM N1 <N2 N3 ...>

Examples: _________

.SUBCKT OPAMP 1 2 3 4

A circuit definition is begun with a .SUBCKT line.  SUBNAM

is the subcircuit name, and N1, N2, ... are the external nodes,

which cannot be zero.  The group of element lines which

immediately follow the .SUBCKT line define the subcircuit.  The

last line in a subcircuit definition is the .ENDS line (see

below).  Control lines may not appear within a subcircuit

definition; however, subcircuit definitions may contain anything

else, including other subcircuit definitions, device models, and

subcircuit calls (see below).  Note that any device models or

subcircuit definitions included as part of a subcircuit

definition are strictly local (i.e., such models and definitions

are not known outside the subcircuit definition).  Also, any

element nodes not included on the .SUBCKT line are strictly

local, with the exception of 0 (ground) which is always global.

8.2.  .ENDS LINE ____  _____ ____

General Form: _______ _____

.ENDS <SUBNAM>

Examples: _________

.ENDS OPAMP

This line must be the last one for any subcircuit

definition.  The subcircuit name, if included, indicates which

subcircuit definition is being terminated; if omitted, all

subcircuits being defined are terminated.  The name is needed

only when nested subcircuit definitions are being made.

8.3.  SUBCIRCUIT CALLS ____  __________ _____

General Form: _______ _____

XYYYYYYY N1 <N2 N3 ...> SUBNAM

Examples: _________

X1 2 4 17 3 1 MULTI

25

Subcircuits are used in SPICE by specifying pseudo-elements

beginning with the letter X, followed by the circuit nodes to be

used in expanding the subcircuit.

9.  CONTROL LINES __  _______ _____

9.1.  .TEMP LINE ____  _____ ____

General Form: _______ _____

.TEMP T1 <T2 <T3 ...>>

Examples: _________

.TEMP -55.0 25.0 125.0

This line specifies the temperatures at which the circuit is

to be simulated.  T1, T2, ... are the different temperatures, in

degrees C.  Temperatures less than -223.0 deg C are ignored.

Model data are specified at TNOM degrees (see the .OPTIONS line

for TNOM); if the .TEMP line is omitted, the simulation will also

be performed at a temperature equal to TNOM.

9.2.  .WIDTH LINE ____  ______ ____

General Form: _______ _____

.WIDTH IN=COLNUM OUT=COLNUM

Examples: _________

.WIDTH IN=72 OUT=133

COLNUM is the last column read from each line of input; the

setting takes effect with the next line read.  The default value

for COLNUM is 80.  The OUT parameter specifies the output print

width.  Permissible values for the output print width are 80 and

133.

9.3.  .OPTIONS LINE ____  ________ ____

General Form: _______ _____

.OPTIONS OPT1 OPT2 ... (OR OPT=OPTVAL ...)

Examples: _________

.OPTIONS ACCT LIST NODE

This line allows the user to reset program control and user

options for specific simulation purposes.  Any combination of the

26

following options may be included, in any order.  'X' (below)

represents some positive number.

OPTION      EFFECT

ACCT        CAUSES ACCOUNTING AND RUN TIME STATISTICS TO BE PRINTED

LIST        CAUSES AN INPUT DATA SUMMARY LISTING TO BE PRINTED

NOMOD       SUPPRESSES THE PRINTOUT OF THE MODEL PARAMETERS.

NOPAGE      SUPPRESSES PAGE EJECTS

NODE        CAUSES THE PRINTING OF THE NODE TABLE.

OPTS        CAUSES THE OPTION VALUES TO BE PRINTED.

GMIN=X      RESETS THE VALUE OF GMIN, THE MINIMUM CONDUCTANCE

ALLOWED BY THE PROGRAM.  THE DEFAULT VALUE IS 1.0E-12.

RELTOL=X    RESETS THE RELATIVE ERROR TOLERANCE OF THE PROGRAM.

THE DEFAULT VALUE IS 0.001 (0.1 PERCENT).

ABSTOL=X    RESETS THE ABSOLUTE CURRENT ERROR TOLERANCE OF THE

PROGRAM.  THE DEFAULT VALUE IS 1 PICOAMP.

VNTOL=X     RESETS THE ABSOLUTE VOLTAGE ERROR TOLERANCE OF THE

PROGRAM.  THE DEFAULT VALUE IS 1 MICROVOLT.

TRTOL=X     RESETS THE TRANSIENT ERROR TOLERANCE.  THE DEFAULT

IS 7.0.  THIS PARAMETER IS AN ESTIMATE OF THE FACTOR

BY WHICH SPICE OVERESTIMATES THE REAL TRUNCATION ERROR.

CHGTOL=X    RESETS THE CHARGE TOLERANCE OF THE PROGRAM.  THE

DEFAULT VALUE IS 1.0E-14.

PIVTOL=X    RESETS THE ABSOLUTE MINIMUM VALUE FOR A MATRIX ENTRY TO

BE ACCEPTED AS A PIVOT.  THE DEFAULT VALUE IS 1.0E-13.

PIVREL=X    RESETS THE RELATIVE RATIO BETWEEN THE LARGEST MATRIX

ENTRY AND AN ACCEPTABLE PIVOT VALUE. THE DEFAULT VALUE

IS 0.001 IN THE NUMERICAL PIVOTING ALGORITHM THE

ALLOWED MINIMUM PIVOT VALUE IS DETERMINED BY

EPSREL=AMAX1(PIVREL*MAXVAL,PIVTOL)

WHERE MAXVAL IS THE MAXIMUM ELEMENT IN THE MATRIX.

NUMDGT=X    RESETS THE NUMBER OF SIGNIFICANT DIGITS PRINTED FOR

OUTPUT VARIABLE VALUES.  X MUST SATISFY THE RELATION

0 < X < 8. THE DEFAULT VALUE IS 4. NOTE: THIS OPTION IS

INDEPENDENT OF THE ERROR TOLERANCE USED (I.E., IF THE

VALUES OF OPTIONS RELTOL, ABSTOL, ETC., ARE UNCHANGED

THEN ONE MAY PRINT NUMERICAL 'NOISE' FOR NUMDGT GT 4.

TNOM=X      RESETS THE NOMINAL TEMPERATURE.  THE DEFAULT VALUE IS

27 DEG C (300 DEG K).

ITL1=X      RESETS THE DC ITERATION LIMIT.  THE DEFAULT IS 100.

ITL2=X      RESETS THE DC TRANSFER CURVE ITERATION LIMIT.  THE

DEFAULT IS 50.

ITL3=X      RESETS THE LOWER TRANSIENT ANALYSIS ITERATION LIMIT.

THE DEFAULT VALUE IS 4.

ITL4=X      RESETS THE TRANSIENT ANALYSIS TIMEPOINT ITERATION

LIMIT.  THE DEFAULT IS 10.

ITL5=X      RESETS THE TRANSIENT ANALYSIS TOTAL ITERATION LIMIT.

THE DEFAULT IS 5000.  SET ITL5=0 TO OMIT THIS TEST.

ITL6=X      RESETS THE DC ITERATION LIMIT AT EACH STEP OF THE

SOURCE STEPPING METHOD.  THE DEFAULT IS 0 WHICH MEANS

27

NOT TO USE THIS METHOD.

CPTIME=X    THE MAXIMUM CPU-TIME IN SECONDS ALLOWED FOR THIS JOB.

LIMTIM=X    RESETS THE AMOUNT OF CPU TIME RESERVED BY SPICE FOR

GENERATING PLOTS SHOULD A CPU TIME-LIMIT CAUSE JOB

TERMINATION.  THE DEFAULT VALUE IS 2 (SECONDS).

LIMPTS=X    RESETS THE TOTAL NUMBER OF POINTS THAT CAN BE PRINTED

OR PLOTTED IN A DC, AC, OR TRANSIENT ANALYSIS.  THE

DEFAULT VALUE IS 201.

LVLCOD=X    IF X IS 2 (TWO), THEN MACHINE CODE FOR THE MATRIX

SOLUTION WILL BE GENERATED.  OTHERWISE, NO MACHINE

CODE IS GENERATED.  THE DEFAULT VALUE IS 2.  APPLIES

ONLY TO CDC COMPUTERS.

LVLTIM=X    IF X IS 1, THE ITERATION TIMESTEP CONTROL IS USED.

IF X IS 2, THE TRUNCATION-ERROR TIMESTEP IS USED.

THE DEFAULT VALUE IS 2. IF METHOD=GEAR AND MAXORD GT 2

THEN LVLTIM IS SET TO 2 BY SPICE.

METHOD=NAME SETS THE NUMERICAL INTEGRATION METHOD USED BY SPICE.

POSSIBLE NAMES ARE GEAR OR TRAPEZOIDAL.  THE DEFAULT

IS TRAPEZOIDAL.

MAXORD=X    SETS THE MAXIMUM ORDER FOR THE INTEGRATION METHOD IF

GEAR'S VARIABLE-ORDER METHOD IS USED.  X MUST BE

BETWEEN 2 AND 6.  THE DEFAULT VALUE IS 2.

DEFL=X      RESETS THE VALUE FOR MOS CHANNEL LENGTH; THE DEFAULT

IS 100.0 MICROMETER.

DEFW=X      RESETS THE VALUE FOR MOS CHANNEL WIDTH; THE DEFAULT

IS 100.0 MICROMETER.

DEFAD=X     RESETS THE VALUE FOR MOS DRAIN DIFFUSION AREA; THE

DEFAULT IS 0.0.

DEFAS=X     RESETS THE VALUE FOR MOS SOURCE DIFFUSION AREA; THE

DEFAULT IS 0.0.

9.4.  .OP LINE ____  ___ ____

General Form: _______ _____

.OP

The inclusion of this line in an input file will force SPICE

to determine the dc operating point of the circuit with inductors

shorted and capacitors opened.  Note: a dc analysis is

automatically performed prior to a transient analysis to

determine the transient initial conditions, and prior to an ac

small-signal analysis to determine the linearized, small-signal

models for nonlinear devices.

SPICE performs a dc operating point analysis if no other

analyses are requested.

9.5.  .DC LINE ____  ___ ____

General Form: _______ _____

28

.DC SRCNAM VSTART VSTOP VINCR <SRC2 START2 STOP2 INCR2>

29

Examples: _________

.DC VIN 0.25 5.0 0.25

.DC VDS 0 10 .5 VGS 0 5 1

.DC VCE 0 10 .25 IB 0 10U 1U

This line defines the dc transfer curve source and sweep

limits.  SRCNAM is the name of an independent voltage or current

source.  VSTART, VSTOP, and VINCR are the starting, final, and

incrementing values respectively.  The first example will cause

the value of the voltage source VIN to be swept from 0.25 volts

to 5.0 volts in increments of 0.25 volts.  A second source (SRC2)

may optionally be specified with associated sweep parameters.  In

this case, the first source will be swept over its range for each

value of the second source.  This option can be useful for

obtaining semiconductor device output characteristics.  See the

second example data set in appendix A.

9.6.  .NODESET LINE ____  ________ ____

General Form: _______ _____

.NODESET V(NODNUM)=VAL V(NODNUM)=VAL ...

Examples: _________

.NODESET V(12)=4.5 V(4)=2.23

This line helps the program find the dc or initial transient

solution by making a preliminary pass with the specified nodes

held to the given voltages.  The restriction is then released and

the iteration continues to the true solution.  The .NODESET line

may be necessary for convergence on bistable or astable circuits.

In general, this line should not be necessary.

9.7.  .IC LINE ____  ___ ____

General Form: _______ _____

.IC V(NODNUM)=VAL V(NODNUM)=VAL ...

Examples: _________

.IC V(11)=5 V(4)=-5 V(2)=2.2

This line is for setting transient initial conditions.  It

has two different interpretations, depending on whether the UIC

parameter is specified on the .TRAN line.  Also, one should not

confuse this line with the .NODESET line.  The .NODESET line is

only to help dc convergence, and does not affect final bias

solution (except for multi-stable circuits).  The two

interpretations of this line are as follows:

30

1.  When the UIC parameter is specified on the .TRAN line,

then the node voltages specified on the .IC line are

used to compute the capacitor, diode, bjt, jfet, and

mosfet initial conditions.  This is equivalent to

specifying the IC=... parameter on each device line, but

is much more convenient.  The IC=... parameter can still

be specified and will take precedence over the .IC

values.  Since no dc bias (initial transient) solution

is computed before the transient analysis, one should

take care to specify all dc source voltages on the .IC

line if they are to be used to compute device initial

conditions.

2.  When the UIC parameter is not specified on the .TRAN

line, the dc bias (initial transient) solution will be

computed before the transient analysis.  In this case,

the node voltages specified on the .IC line will be

forced to the desired initial values during the bias

solution.  During transient analysis, the constraint on

these node voltages is removed.

9.8.  .TF LINE ____  ___ ____

General Form: _______ _____

.TF OUTVAR INSRC

Examples: _________

.TF V(5,3) VIN

.TF I(VLOAD) VIN

This line defines the small-signal output and input for the

dc small-signal analysis.  OUTVAR is the small-signal output

variable and INSRC is the small-signal input source.  If this

line is included, SPICE will compute the dc small-signal value of

the transfer function (output/input), input resistance, and

output resistance.  For the first example, SPICE would compute

the ratio of V(5,3) to VIN, the small-signal input resistance at

VIN, and the small-signal output resistance measured across nodes

5 and 3.

9.9.  .SENS LINE ____  _____ ____

General Form: _______ _____

.SENS OV1 <OV2 ... >

Examples: _________

.SENS V(9) V(4,3) V(17) I(VCC)

31

If a .SENS line is included in the input file, SPICE will

determine the dc small-signal sensitivities of each specified

output variable with respect to every circuit parameter.  NOTE:                                                           _____

for large circuits, large amounts of output can be generated.

9.10.  .AC LINE _____  ___ ____

General Form: _______ _____

.AC DEC ND FSTART FSTOP

.AC OCT NO FSTART FSTOP

.AC LIN NP FSTART FSTOP

Examples: _________

.AC DEC 10 1 10K

.AC DEC 10 1K 100MEG

.AC LIN 100 1 100HZ

DEC stands for decade variation, and ND is the number of

points per decade.  OCT stands for octave variation, and NO is

the number of points per octave.  LIN stands for linear variation

and NP is the number of points.  FSTART is the starting

frequency, and FSTOP is the final frequency.  If this line is

included in the file, SPICE will perform an ac analysis of the

circuit over the specified frequency range.  Note that in order

for this analysis to be meaningful, at least one independent

source must have been specified with an ac value.

9.11.  .DISTO LINE _____  ______ ____

General Form: _______ _____

.DISTO RLOAD <INTER <SKW2 <REFPWR <SPW2>>>>

Examples: _________

.DISTO RL 2 0.95 1.0E-3 0.75

This line controls whether SPICE will compute the distortion

characteristic of the circuit in a small-signal mode as a part of

the ac small-signal sinusoidal steady-state analysis.  The

analysis is performed assuming that one or two signal frequencies

are imposed at the input; let the two frequencies be F1 (the

nominal analysis frequency) and F2 (=SKW2*F1).  The program then

computes the following distortion measures:

1.  HD2 - the magnitude of the frequency component 2*F1

assuming that F2 is not present.

2.  HD3 - the magnitude of the frequency component 3*F1

assuming that F2 is not present.

32

3.  SIM2 - the magnitude of the frequency component F1 + F2.

4.  DIM2 - the magnitude of the frequency component F1 - F2.

5.  DIM3 - the magnitude of the frequency component 2*F1 -

F2.

RLOAD is the name of the output load resistor into which all

distortion power products are to be computed.  INTER is the

interval at which the summary printout of the contributions of

all nonlinear devices to the total distortion is to be printed.

If omitted or set to zero, no summary printout will be made.

REFPWR is the reference power level used in computing the

distortion products; if omitted, a value of 1 mW (that is, 0 dBm)

is used.  SKW2 is the ratio of F2 to F1.  If omitted, a value of

0.9 is used (i.e., F2 = 0.9*F1).  SPW2 is the amplitude of F2.

If omitted, a value of 1.0 is assumed.

The distortion measures HD2, HD3, SIM2, DIM2, and DIM3 may

also be printed and/or plotted (see the description of the .PRINT

and .PLOT lines).

9.12.  .NOISE LINE _____  ______ ____

General Form: _______ _____

.NOISE OUTV INSRC NUMS

Examples: _________

.NOISE V(5) VIN 10

This line controls the noise analysis of the circuit.  The

noise analysis is performed in conjunction with the ac analysis

(see .AC line).  OUTV is an output voltage which defines the

summing point.  INSRC is the name of the independent voltage or

current source which is the noise input reference.  NUMS is the

summary interval.  SPICE will compute the equivalent output noise

at the specified output as well as the equivalent input noise at

the specified input.  In addition, the contributions of every

noise generator in the circuit will be printed at every NUMS

frequency points (the summary interval).  If NUMS is zero, no

summary printout will be made.

The output noise and the equivalent input noise may also be

printed and/or plotted (see the description of the .PRINT and

.PLOT lines).

33

9.13.  .TRAN LINE _____  _____ ____

General Form: _______ _____

.TRAN TSTEP TSTOP <TSTART <TMAX>> <UIC>

Examples: _________

.TRAN 1NS 100NS

.TRAN 1NS 1000NS 500NS

.TRAN 10NS 1US UIC

TSTEP is the printing or plotting increment for line-printer

output.

For use with the post-processor, TSTEP is the suggested

computing increment.

TSTOP is the final time, and TSTART is the initial time.  If

TSTART is omitted, it is assumed to be zero.  The transient

analysis always begins at time zero.  In the interval <ZERO,

TSTART>, the circuit is analyzed (to reach a steady state), but

no outputs are stored.  In the interval <TSTART, TSTOP>, the

circuit is analyzed and outputs are stored.  TMAX is the maximum

stepsize that SPICE will use (for default, the program chooses

either TSTEP or (TSTOP-TSTART)/50.0, whichever is smaller.  TMAX

is useful when one wishes to guarantee a computing interval which

is smaller than the printer increment, TSTEP.

UIC (use initial conditions) is an optional keyword which

indicates that the user does not want SPICE to solve for the

quiescent operating point before beginning the transient

analysis.  If this keyword is specified, SPICE uses the values

specified using IC=... on the various elements as the initial

transient condition and proceeds with the analysis.  If the .IC

line has been specified, then the node voltages on the .IC line

are used to compute the initial conditions for the devices.  Look

at the description on the .IC line for its interpretation when

UIC is not specified.

9.14.  .FOUR LINE _____  _____ ____

General Form: _______ _____

.FOUR FREQ OV1 <OV2 OV3 ...>

Examples: _________

.FOUR 100K  V(5)

This line controls whether SPICE performs a Fourier analysis

as a part of the transient analysis.  FREQ is the fundamental

34

frequency, and OV1, ..., are the output variables for which the

analysis is desired.  The Fourier analysis is performed over the

interval <TSTOP-PERIOD, TSTOP>, where TSTOP is the final time

specified for the transient analysis, and PERIOD is one period of

the fundamental frequency.  The dc component and the first nine

components are determined.  For maximum accuracy, TMAX (see the

.TRAN line) should be set to PERIOD/100.0 (or less for very

high-Q circuits).

9.15.  .PRINT LINES _____  ______ _____

General Form: _______ _____

.PRINT PRTYPE OV1 <OV2 ... OV8>

Examples: _________

.PRINT TRAN V(4) I(VIN)

.PRINT AC VM(4,2) VR(7) VP(8,3)

.PRINT DC V(2) I(VSRC) V(23,17)

.PRINT NOISE INOISE

.PRINT DISTO HD3 SIM2(DB)

This line defines the contents of a tabular listing of one

to eight output variables.  PRTYPE is the type of the analysis

(DC, AC, TRAN, NOISE, or DISTO) for which the specified outputs

are desired.  The form for voltage or current output variables is

as follows: V(N1<,N2>) specifies the voltage difference between

nodes N1 and N2.  if N2 (and the preceding comma) is omitted,

ground (0) is assumed.  For the ac analysis, five additional

outputs can be accessed by replacing the letter V by:

VR  -    REAL PART

VI  -    IMAGINARY PART

VM  -    MAGNITUDE

VP  -    PHASE

VDB -    20*LOG10(MAGNITUDE)

I(VXXXXXXX) specifies the current flowing in the independent

voltage source named VXXXXXXX.  Positive current flows from the

positive node, through the source, to the negative node.  For the

ac analysis, the corresponding replacements for the letter I may

be made in the same way as described for voltage outputs.

Output variables for the noise and distortion analyses have

a different general form from that of the other analyses, i.e.

OV<(X)>

where OV is any of ONOISE (output noise), INOISE (equivalent

input noise),D2, HD3, SIM2, DIM2, or DIM3 (see description of

distortion analysis), and X may be any of:

35

R  -    REAL PART

I  -    IMAGINARY PART

M  -    MAGNITUDE (DEFAULT IF NOTHING SPECIFIED)

P  -    PHASE

DB -    20*LOG10(MAGNITUDE)

Thus, SIM2 (or SIM2(M)) describes the magnitude of the SIM2

distortion measure, while HD2(R) describes the real part of the

HD2 distortion measure.  There is no limit on the number of

.PRINT lines for each type of analysis.

9.16.  .PLOT LINES _____  _____ _____

General Form: _______ _____

.PLOT PLTYPE OV1 <(PLO1,PHI1)> <OV2 <(PLO2,PHI2)> ... OV8>

Examples: _________

.PLOT DC V(4) V(5) V(1)

.PLOT TRAN V(17,5) (2,5) I(VIN) V(17) (1,9)

.PLOT AC VM(5) VM(31,24) VDB(5) VP(5)

.PLOT DISTO HD2 HD3(R) SIM2

.PLOT TRAN V(5,3) V(4) (0,5) V(7) (0,10)

This line defines the contents of one plot of from one to

eight output variables.  PLTYPE is the type of analysis (DC, AC,

TRAN, NOISE, or DISTO) for which the specified outputs are

desired.  The syntax for the OVI is identical to that for the

.PRINT line, described above.

The optional plot limits (PLO,PHI) may be specified after

any of the output variables.  All output variables to the left of

a pair of plot limits (PLO,PHI) will be plotted using the same

lower and upper plot bounds.  If plot limits are not specified,

SPICE will automatically determine the minimum and maximum values

of all output variables being plotted and scale the plot to fit.

More than one scale will be used if the output variable values

warrant (i.e., mixing output variables with values which are

orders-of-magnitude different still gives readable plots).

The overlap of two or more traces on any plot is indicated

by the letter X.

When more than one output variable appears on the same plot,

the first variable specified will be printed as well as plotted.

If a printout of all variables is desired, then a companion

.PRINT line should be included.  There is no limit on the number

of .PLOT lines specified for each type of analysis.

[1] A. Vladimirescu and S. Liu, "The Simulation of MOS Integrated

36

Circuits Using SPICE2", ERL memo No. ERL M80/7, Electronics

Research Laboratory, University of California, Berkeley, Oct.,

1980.

37

10.  APPENDIX A: EXAMPLE DATA SETS ___  ________ __ _______ ____ ____

10.1.  CIRCUIT 1 _____  _______ _

The following data set determines the dc operating point and

small-signal transfer function of a simple differential pair.  In

addition, the ac small-signal response is computed over the

frequency range 1 Hz to 100 MHz.

SIMPLE DIFFERENTIAL PAIR

VCC 7 0 12

VEE 8 0 -12

VIN 1 0 AC 1

RS1 1 2 1K

RS2 6 0 1K

Q1 3 2 4 MOD1

Q2 5 6 4 MOD1

RC1 7 3 10K

RC2 7 5 10K

RE 4 8 10K

.MODEL MOD1 NPN BF=50 VAF=50 IS=1.E-12 RB=100 CJC=.5PF TF=.6NS

.TF V(5) VIN

.AC DEC 10 1 100MEG

.PLOT AC VM(5) VP(5)

.PRINT AC VM(5) VP(5)

.END

10.2.  CIRCUIT 2 _____  _______ _

The following data set computes the output characteristics

of a mosfet device over the range 0-10 V for VDS and 0-5 V for

VGS.

MOS OUTPUT CHARACTERISTICS

.OPTIONS NODE NOPAGE

VDS 3 0

VGS 2 0

M1 1 2 0 0 MOD1 L=4U W=6U AD=10P AS=10P

.MODEL MOD1 NMOS VTO=-2 NSUB=1.0E15 UO=550

* VIDS MEASURES ID, WE COULD HAVE USED VDS,

* BUT ID WOULD BE NEGATIVE

VIDS 3 1

.DC VDS 0 10 .5 VGS 0 5 1

.PRINT DC I(VIDS) V(2)

.PLOT DC I(VIDS)

.END

38

10.3.  CIRCUIT 3 _____  _______ _

The following data set determines the dc transfer curve and

the transient pulse response of a simple RTL inverter.  The input

is a pulse from 0 to 5 volts with delay, rise, and fall times of

2 ns and a pulse width of 30 ns.  The transient interval is 0 to

100 ns, with printing to be done every nanosecond.

SIMPLE RTL INVERTER

VCC 4 0 5

VIN 1 0 PULSE 0 5 2NS 2NS 2NS 30NS

RB 1 2 10K

Q1 3 2 0 Q1

RC 3 4 1K

.PLOT DC V(3)

.PLOT TRAN V(3) (0,5)

.PRINT TRAN V(3)

.MODEL Q1 NPN BF 20 RB 100 TF .1NS CJC 2PF

.DC VIN 0 5 0.1

.TRAN 1NS 100NS

.END

10.4.  CIRCUIT 4 _____  _______ _

The following data set simulates a four-bit binary adder,

using several subcircuits to describe various pieces of the

overall circuit.

ADDER - 4 BIT ALL-NAND-GATE BINARY ADDER

*** SUBCIRCUIT DEFINITIONS

.SUBCKT NAND 1 2 3 4

*   NODES:  INPUT(2), OUTPUT, VCC

Q1 9 5 1 QMOD

D1CLAMP 0 1 DMOD

Q2 9 5 2 QMOD

D2CLAMP 0 2 DMOD

RB 4 5 4K

R1 4 6 1.6K

Q3 6 9 8 QMOD

R2 8 0 1K

RC 4 7 130

Q4 7 6 10 QMOD

DVBEDROP 10 3 DMOD

Q5 3 8 0 QMOD

.ENDS NAND

.SUBCKT ONEBIT 1 2 3 4 5 6

*   NODES:  INPUT(2), CARRY-IN, OUTPUT, CARRY-OUT, VCC

X1 1 2 7 6 NAND

X2 1 7 8 6 NAND

X3 2 7 9 6 NAND

X4 8 9 10 6 NAND

X5 3 10 11 6 NAND

39

X6 3 11 12 6 NAND

X7 10 11 13 6 NAND

X8 12 13 4 6 NAND

X9 11 7 5 6 NAND

.ENDS ONEBIT

.SUBCKT TWOBIT 1 2 3 4 5 6 7 8 9

*   NODES:  INPUT - BIT0(2) / BIT1(2), OUTPUT - BIT0 / BIT1,

*           CARRY-IN, CARRY-OUT, VCC

X1 1 2 7 5 10 9 ONEBIT

X2 3 4 10 6 8 9 ONEBIT

.ENDS TWOBIT

.SUBCKT FOURBIT 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15

*   NODES: INPUT - BIT0(2) / BIT1(2) / BIT2(2) / BIT3(2),

*          OUTPUT - BIT0/BIT1/BIT2/BIT3,CARRY-IN,CARRY-OUT,VCC

X1 1 2 3 4 9 10 13 16 15 TWOBIT

X2 5 6 7 8 11 12 16 14 15 TWOBIT

.ENDS FOURBIT

*** DEFINE NOMINAL CIRCUIT

.MODEL DMOD D

.MODEL QMOD NPN(BF=75 RB=100 CJE=1PF CJC=3PF)

VCC 99 0 DC 5V

VIN1A 1 0 PULSE(0 3 0 10NS 10NS   10NS   50NS)

VIN1B 2 0 PULSE(0 3 0 10NS 10NS   20NS  100NS)

VIN2A 3 0 PULSE(0 3 0 10NS 10NS   40NS  200NS)

VIN2B 4 0 PULSE(0 3 0 10NS 10NS   80NS  400NS)

VIN3A 5 0 PULSE(0 3 0 10NS 10NS  160NS  800NS)

VIN3B 6 0 PULSE(0 3 0 10NS 10NS  320NS 1600NS)

VIN4A 7 0 PULSE(0 3 0 10NS 10NS  640NS 3200NS)

VIN4B 8 0 PULSE(0 3 0 10NS 10NS 1280NS 6400NS)

X1 1 2 3 4 5 6 7 8 9 10 11 12 0 13 99 FOURBIT

RBIT0 9 0 1K

RBIT1 10 0 1K

RBIT2 11 0 1K

RBIT3 12 0 1K

RCOUT 13 0 1K

.PLOT TRAN V(1) V(2) V(3) V(4) V(5) V(6) V(7) V(8)

.PLOT TRAN V(9) V(10) V(11) V(12) V(13)

.PRINT TRAN V(1) V(2) V(3) V(4) V(5) V(6) V(7) V(8)

.PRINT TRAN V(9) V(10) V(11) V(12) V(13)

*** (FOR THOSE WITH MONEY (AND MEMORY) TO BURN)

.TRAN 1NS 6400NS

.OPTIONS ACCT LIST NODE LIMPTS=6401

.END

40

10.5.  CIRCUIT 5 _____  _______ _

The following data set simulates a transmission-line

inverter.  Two transmission-line elements are required since two

propagation modes are excited.  In the case of a coaxial line,

the first line (T1) models the inner conductor with respect to

the shield, and the second line (T2) models the shield with

respect to the outside world.

TRANSMISSION-LINE INVERTER

V1 1 0 PULSE(0 1 0 0.1N)

R1 1 2 50

X1 2 0 0 4 TLINE

R2 4 0 50

.SUBCKT TLINE 1 2 3 4

T1 1 2 3 4 Z0=50 TD=1.5NS

T2 2 0 4 0 Z0=100 TD=1NS

.ENDS TLINE

.TRAN 0.1NS 20NS

.PLOT TRAN V(2) V(4)

.END

41

11.  APPENDIX B: NONLINEAR DEPENDENT SOURCES ___  ________ __ _________ _________ _______

SPICE allows circuits to contain dependent sources

characterized by any of the four equations

I=F(V)          V=F(V)          I=F(I)          V=F(I)

where the functions must be polynomials, and the arguments may be

multidimensional.  The polynomial functions are specified by a

set of coefficients P0, P1, ..., PN.  Both the number of

dimensions and the number of coefficients are arbitrary.  The

meaning of the coefficients depends upon the dimension of the

polynomial, as shown in the following examples:

Suppose that the function is one-dimensional (that is, a

function of one argument).  Then the function value FV is

determined by the following expression in FA (the function

argument):

FV = P0 + (P1*FA) + (P2*FA**2) + (P3*FA**3) + (P4*FA**4)

+ (P5*FA**5) + ...

Suppose now that the function is two-dimensional, with

arguments FA and FB.  Then the function value FV is determined by

the following expression:

FV = P0 + (P1*FA) + (P2*FB) + (P3*FA**2) + (P4*FA*FB)

+ (P5*FB**2) + (P6*FA**3) + (P7*FA**2*FB) + (P8*FA*FB**2)

+ (P9*FB**3) + ...

Consider now the case of a three-dimensional polynomial

function with arguments FA, FB, and FC.  Then the function value

FV is determined by the following expression:

FV = P0 + (P1*FA) + (P2*FB) + (P3*FC) + (P4*FA**2) + (P5*FA*FB)

+ (P6*FA*FC) + (P7*FB**2) + (P8*FB*FC) + (P9*FC**2)

+ (P10*FA**3) + (P11*FA**2*FB) + (P12*FA**2*FC) + (P13*FA*FB**2)

+ (P14*FA*FB*FC) + (P15*FA*FC**2) + (P16*FB**3) + (P17*FB**2*FC)

+ (P18*FB*FC**2) + (P19*FC**3) + (P20*FA**4) + ...

NOTE: if the polynomial is one-dimensional and exactly one _____

coefficient is specified, then SPICE assumes it to be P1 (and P0

= 0.0), in order to facilitate the input of linear controlled

sources.

42

For all four of the dependent source types described below,

the initial condition parameter is described as optional.  If not

specified, SPICE assumes 0.0.  The initial condition for

dependent sources is an initial 'guess' for the value of the

controlling variable.  The program uses this initial condition to

obtain the dc operating point of the circuit.  After convergence

has been obtained, the program continues iterating to obtain the

exact value for the controlling variable.  Hence, to reduce the

computational effort for the dc operating point (or if the

polynomial specifies a strong nonlinearity), a value fairly close

to the actual controlling variable should be specified for the

initial condition.

11.1.  VOLTAGE-CONTROLLED CURRENT SOURCES _____  __________________ _______ _______

General Form: _______ _____

GXXXXXXX N+ N- <POLY(ND)> NC1+ NC1- ... P0 <P1 ...> <IC=...>

Examples: _________

G1 1 0 5 3 0 0.1M

GR 17 3 17 3 0 1M 1.5M IC=2V

GMLT 23 17 POLY(2) 3 5 1 2 0 1M 17M 3.5U IC=2.5, 1.3

N+ and N- are the positive and negative nodes, respectively.

Current flow is from the positive node, through the source, to

the negative node.  POLY(ND) only has to be specified if the

source is multi-dimensional (one-dimensional is the default).  If

specified, ND is the number of dimensions, which must be

positive.  NC1+, NC1-, ... are the positive and negative

controlling nodes, respectively.  One pair of nodes must be

specified for each dimension.  P0, P1, P2, ..., PN are the

polynomial coefficients.  The (optional) initial condition is the

initial guess at the value(s) of the controlling voltage(s).  If

not specified, 0.0 is assumed.  The polynomial specifies the

source current as a function of the controlling voltage(s).  The

second example above describes a current source with value

I = 1E-3*V(17,3) + 1.5E-3*V(17,3)**2

Note that since the source nodes are the same as the controlling

nodes, this source actually models a nonlinear resistor.

11.2.  VOLTAGE-CONTROLLED VOLTAGE SOURCES _____  __________________ _______ _______

General Form: _______ _____

EXXXXXXX N+ N- <POLY(ND)> NC1+ NC1- ... P0 <P1 ...> <IC=...>

43

Examples: _________

E1 3 4 21 17 10.5 2.1 1.75

EX 17 0 POLY(3) 13 0 15 0 17 0 0 1 1 1 IC=1.5,2.0,17.35

N+ and N- are the positive and negative nodes, respectively.

POLY(ND) only has to be specified if the source is

multi-dimensional (one-dimensional is the default).  If

specified, ND is the number of dimensions, which must be

positive.  NC1+, NC1-, ... are the positive and negative

controlling nodes, respectively.  One pair of nodes must be

specified for each dimension.  P0, P1, P2, ..., PN are the

polynomial coefficients.  The (optional) initial condition is the

initial guess at the value(s) of the controlling voltage(s).  If

not specified, 0.0 is assumed.  The polynomial specifies the

source voltage as a function of the controlling voltage(s).  The

second example above describes a voltage source with value

V = V(13,0) + V(15,0) + V(17,0)

(In other words, an ideal voltage summer).

11.3.  CURRENT-CONTROLLED CURRENT SOURCES _____  __________________ _______ _______

General Form: _______ _____

FXXXXXXX N+ N- <POLY(ND)> VN1 <VN2 ...> P0 <P1 ...> <IC=...>

Examples: _________

F1 12 10 VCC 1MA 1.3M

FXFER 13 20 VSENS 0 1

N+ and N- are the positive and negative nodes, respectively.

Current flow is from the positive node, through the source, to

the negative node.  POLY(ND) only has to be specified if the

source is multi-dimensional (one-dimensional is the default).  If

specified, ND is the number of dimensions, which must be

positive.  VN1, VN2, ... are the names of voltage sources through

which the controlling current flows; one name must be specified

for each dimension.  The direction of positive controlling

current flow is from the positive node, through the source, to

the negative node of each voltage source.  P0, P1, P2, ..., PN

are the polynomial coefficients.  The (optional) initial

condition is the initial guess at the value(s) of the controlling

current(s) (in amps).  If not specified, 0.0 is assumed.  The

polynomial specifies the source current as a function of the

controlling current(s).  The first example above describes a

current source with value

I = 1E-3 + 1.3E-3*I(VCC)

44

11.4.  CURRENT-CONTROLLED VOLTAGE SOURCES _____  __________________ _______ _______

General Form: _______ _____

HXXXXXXX N+ N- <POLY(ND)> VN1 <VN2 ...> P0 <P1 ...> <IC=...>

Examples: _________

HXY 13 20 POLY(2) VIN1 VIN2 0 0 0 0 1 IC=0.5 1.3

HR 4 17 VX 0 0 1

N+ and N- are the positive and negative nodes, respectively.

POLY(ND) only has to be specified if the source is

multi-dimensional (one-dimensional is the default).  If

specified, ND is the number of dimensions, which must be

positive.  VN1, VN2, ... are the names of voltage sources through

which the controlling current flows; one name must be specified

for each dimension.  The direction of positive controlling

current flow is from the positive node, through the source, to

the negative node of each voltage source.  P0, P1, P2, ..., PN

are the polynomial coefficients.  The (optional) initial

condition is the initial guess at the value(s) of the controlling

current(s) (in amps).  If not specified, 0.0 is assumed.  The

polynomial specifies the source voltage as a function of the

controlling current(s).  The first example above describes a

voltage source with value

V = I(VIN1)*I(VIN2)

45

12.  APPENDIX D: ALTER STATEMENT AND THE SOURCE-STEPPING METHOD ___  ________ __ _____ _________ ___ ___ _______________ ______

The .ALTER statement allows SPICE to run with altered

circuit parameters.

General Form: _______ _____

.ALTER

ELEMENT LINES (DEVICE LINES, MODEL LINES)

.ALTER (OR .END LINE)

Examples: _________

R1 1 0 5K

VCC 3 0 10

M1 3 2 0 MOD1 L=5U W=2U

.MODEL MOD1 NMOS(VTO=1.0 KP=2.0E-5 PHI=0.6 NSUB=2.0E15 TOX=0.1U)

.ALTER

R1 1 0 3.5K

VCC 3 0 12

M1 3 2 0 MOD1 L=10U W=2U

.MODEL MOD1 NMOS(VTO=1.2 KP=2.0E-5 PHI=0.6 NSUB=5.0E15 TOX=1.5U)

.ALTER

M1 3 2 0 MOD1 L=10U W=4U

.END

This line introduces the element(s), device(s) and model(s)

whose parameters are changed during the execution of the input

data set.  the analyses specified in the data set will start over

again with the changed parameters.  The .ALTER line with the

lines defining the new parameters should be placed just before

the .END line.  The syntax for the element (device, model) lines

is identical to that of the lines with the original parameters.

There is no limit on the number of .ALTER lines and the

circuit will be re-analyzed as many times as the number of .ALTER

lines.  Subsequent ALTER operations employ parameters of the

previous change.  No topological change of the circuit is                   __ ___________ ______ __ ___ _______ __

allowed.  ________

The source-stepping method can enhance dc convergence.  But

it is slower than direct use of the Newton-Raphson method.

Therefore it is best used as an alternative to achieve

convergence of dc operating point when the circuit fails to

converge by using the Newton-Raphson method.  The source-stepping

method is used by SPICE when the variable ITL6 in the .OPTIONS

line is set to the iteration limit at each step of the source(s).

For example,

.OPTIONS ITL6=30

46

will cause SPICE to use source-stepping method with iteration

limit 30 at each step.  By default, ITL6 is 0 which means to use

the Newton-Raphson method directly.